Day2：服务器架构核心（NUMA + PCIe + 网络）

1、NUMA架构（服务器核心概念）

Non-Uniform Memory Access非统一内存访问

不同CPU访问不同内存的速度不一样

CPU0 ── 本地内存

│

└─ 访问很快

CPU1 ── 本地内存

│

└─ 访问很快

CPU0 访问 CPU1 内存

│

└─ 访问较慢

CPU + 内存 是绑定的、这就是内存节点、

[root@bogon ~]# lscpu | grep NUMA

NUMA 节点：                         4

NUMA 节点0 CPU：                    0-63

NUMA 节点1 CPU：                    64-127

NUMA 节点2 CPU：                    128-191

NUMA 节点3 CPU：                    192-255

[root@bogon ~]#

CPU0 → node0 node1

CPU1 → node2 node3

CPU0

├── NUMA0

└── NUMA1

CPU1

├── NUMA2

└── NUMA3

[root@bogon ~]# numactl --hardware

available: 4 nodes (0-3)

node 0 cpus: 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63

node 0 size: 515958 MB

node 0 free: 513966 MB

node 1 cpus: 64 65 66 67 68 69 70 71 72 73 74 75 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 100 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 116 117 118 119 120 121 122 123 124 125 126 127

node 1 size: 515401 MB

node 1 free: 514252 MB

node 2 cpus: 128 129 130 131 132 133 134 135 136 137 138 139 140 141 142 143 144 145 146 147 148 149 150 151 152 153 154 155 156 157 158 159 160 161 162 163 164 165 166 167 168 169 170 171 172 173 174 175 176 177 178 179 180 181 182 183 184 185 186 187 188 189 190 191

node 2 size: 515011 MB

node 2 free: 513672 MB

node 3 cpus: 192 193 194 195 196 197 198 199 200 201 202 203 204 205 206 207 208 209 210 211 212 213 214 215 216 217 218 219 220 221 222 223 224 225 226 227 228 229 230 231 232 233 234 235 236 237 238 239 240 241 242 243 244 245 246 247 248 249 250 251 252 253 254 255

node 3 size: 516086 MB

node 3 free: 514845 MB

node distances:

node   0   1   2   3

0:  10  12  35  37

1:  12  10  37  40

2:  35  37  10  12

3:  37  40  12  10

[root@bogon ~]#

256核、四个内存节点

- 2、CPU与内存拓扑

- [root@bogon ~]# lscpu

- 架构：                  aarch64

- CPU 运行模式：        64-bit

- 字节序：              Little Endian

- CPU:                    256

- 在线 CPU 列表：       0-255

- 厂商 ID：               HiSilicon

- BIOS Vendor ID:       HiSilicon

- BIOS Model name:      Kunpeng 920 7260Z

- 型号：                0

- 每个核的线程数：      2

- 每个座的核数：        64

- 座：                  2

- 步进：                0x0

- Frequency boost:      disabled

- CPU 最大 MHz：        2600.0000

- CPU 最小 MHz：        400.0000

- BogoMIPS：            200.00

- 标记：                fp asimd evtstrm aes pmull sha1 sha2 crc32 atomics fphp asimdhp cpuid asimdrdm jscvt fcma lrcpc dcpop sha3 sm

- 3 sm4 asimddp sha512 sve asimdfhm dit uscat ilrcpc flagm ssbs sb dcpodp flagm2 frint svei8mm svef32mm svef64m

- m svebf16 i8mm bf16 dgh rng ecv

- Caches (sum of all):

- L1d:                  8 MiB (128 instances)

- L1i:                  8 MiB (128 instances)

- L2:                   160 MiB (128 instances)

- L3:                   224 MiB (4 instances)

- NUMA:

- NUMA 节点：           4

- NUMA 节点0 CPU：      0-63

- NUMA 节点1 CPU：      64-127

- NUMA 节点2 CPU：      128-191

- NUMA 节点3 CPU：      192-255

- Vulnerabilities:

- Gather data sampling: Not affected

- Itlb multihit:        Not affected

- L1tf:                 Not affected

- Mds:                  Not affected

- Meltdown:             Not affected

- Mmio stale data:      Not affected

- Retbleed:             Not affected

- Spec rstack overflow: Not affected

- Spec store bypass:    Mitigation; Speculative Store Bypass disabled via prctl

- Spectre v1:           Mitigation; __user pointer sanitization

- Spectre v2:           Not affected

- Srbds:                Not affected

- Tsx async abort:      Not affected

- [root@bogon ~]#

- 3、PCIe链路速度

- [root@bogon ~]# lspci -vv -s 81:00.0 | grep Lnk

- LnkCap: Port #0, Speed 16GT/s, Width x4, ASPM not supported

- LnkCtl: ASPM Disabled; RCB 128 bytes, Disabled- CommClk-

- LnkSta: Speed 16GT/s, Width x4

- LnkCap2: Supported Link Speeds: 2.5-16GT/s, Crosslink- Retimer+ 2Retimers+ DRS-

- LnkCtl2: Target Link Speed: 16GT/s, EnterCompliance- SpeedDis-

- LnkSta2: Current De-emphasis Level: -6dB, EqualizationComplete+ EqualizationPhase1+

- LnkCtl3: LnkEquIntrruptEn- PerformEqu-

- [root@bogon ~]#

- [root@bogon ~]# lspci -vv | grep -i  1515

- 81:00.0 Non-Volatile memory controller: DERA Storage Device 1515 (prog-if 02 [NVM Express])

- 82:00.0 Non-Volatile memory controller: DERA Storage Device 1515 (prog-if 02 [NVM Express])

- [root@bogon ~]# lspci -vv -s 81:00.0

- 81:00.0 Non-Volatile memory controller: DERA Storage Device 1515 (prog-if 02 [NVM Express])

- Subsystem: DERA Storage Device 7105

- Physical Slot: 67

- Control: I/O+ Mem+ BusMaster+ SpecCycle- MemWINV- VGASnoop- ParErr+ Stepping- SERR+ FastB2B- DisINTx+

- Status: Cap+ 66MHz- UDF- FastB2B- ParErr- DEVSEL=fast >TAbort- <TAbort- <MAbort- >SERR- <PERR- INTx-

- Latency: 0, Cache Line Size: 32 bytes

- Interrupt: pin A routed to IRQ 28

- NUMA node: 3  ##NVME挂在NUMA node3、如果程序跑在CPU 0-63访问nvme就会跨NUMA性能就会下降、

- CPU0

- │

- Interconnect

- │

- CPU1

- │

- PCIe

- │

- NVMe

- 多了一段CPU ↔ CPU 互联(4-5GB/S)延迟增加增加 20%~50%

- ####服务器优化CPU绑定、NUMA绑定           ：numactl --cpunodebind=3后CPU → PCIe → NVMe（7GB/S）

- numactl --cpunodebind=3 --membind=3 程序CPU 使用 node3

- 内存 使用 node3    CPU、内存、NVMe都在一个numa

- Region 0: Memory at a0320000 (64-bit, non-prefetchable) [size=32K]

- Expansion ROM at a0300000 [disabled] [size=128K]

- Capabilities: [80] Power Management version 3

- Flags: PMEClk- DSI- D1- D2- AuxCurrent=0mA PME(D0-,D1-,D2-,D3hot-,D3cold-)

- Status: D0 NoSoftRst+ PME-Enable- DSel=0 DScale=0 PME-

- Capabilities: [90] MSI: Enable- Count=1/1 Maskable+ 64bit+

- Address: 0000000000000000  Data: 0000

- Masking: 00000000  Pending: 00000000

- Capabilities: [b0] MSI-X: Enable+ Count=257 Masked-

- Vector table: BAR=0 offset=00004000

- PBA: BAR=0 offset=00006000

- Capabilities: [c0] Express (v2) Endpoint, MSI 00

- DevCap: MaxPayload 512 bytes, PhantFunc 0, Latency L0s <1us, L1 <1us

- ExtTag- AttnBtn- AttnInd- PwrInd- RBE+ FLReset+ SlotPowerLimit 0W

- DevCtl: CorrErr+ NonFatalErr+ FatalErr+ UnsupReq+

- RlxdOrd+ ExtTag- PhantFunc- AuxPwr- NoSnoop+ FLReset-

- MaxPayload 256 bytes, MaxReadReq 512 bytes

- DevSta: CorrErr- NonFatalErr- FatalErr- UnsupReq- AuxPwr- TransPend-

- LnkCap: Port #0, Speed 16GT/s, Width x4, ASPM not supported

- ClockPM- Surprise- LLActRep- BwNot- ASPMOptComp+

- LnkCtl: ASPM Disabled; RCB 128 bytes, Disabled- CommClk-

- ExtSynch- ClockPM- AutWidDis- BWInt- AutBWInt-

- LnkSta: Speed 16GT/s, Width x4###cap和sta数据一致、带宽没有变低、延迟没有变高

- TrErr- Train- SlotClk- DLActive- BWMgmt- ABWMgmt-

- DevCap2: Completion Timeout: Range B, TimeoutDis+ NROPrPrP- LTR+

- 10BitTagComp+ 10BitTagReq- OBFF Via message, ExtFmt+ EETLPPrefix+, MaxEETLPPrefixes 2

- EmergencyPowerReduction Not Supported, EmergencyPowerReductionInit-

- FRS- TPHComp+ ExtTPHComp+

- AtomicOpsCap: 32bit- 64bit- 128bitCAS-

- DevCtl2: Completion Timeout: 65ms to 210ms, TimeoutDis- LTR- 10BitTagReq- OBFF Disabled,

- AtomicOpsCtl: ReqEn-

- LnkCap2: Supported Link Speeds: 2.5-16GT/s, Crosslink- Retimer+ 2Retimers+ DRS-

- LnkCtl2: Target Link Speed: 16GT/s, EnterCompliance- SpeedDis-

- Transmit Margin: Normal Operating Range, EnterModifiedCompliance- ComplianceSOS-

- Compliance Preset/De-emphasis: -6dB de-emphasis, 0dB preshoot

- LnkSta2: Current De-emphasis Level: -6dB, EqualizationComplete+ EqualizationPhase1+

- EqualizationPhase2+ EqualizationPhase3+ LinkEqualizationRequest-

- Retimer- 2Retimers- CrosslinkRes: unsupported

- Capabilities: [100 v2] Advanced Error Reporting

- UESta:  DLP- SDES- TLP- FCP- CmpltTO- CmpltAbrt- UnxCmplt- RxOF- MalfTLP- ECRC- UnsupReq- ACSViol-

- UEMsk:  DLP- SDES- TLP- FCP- CmpltTO- CmpltAbrt- UnxCmplt- RxOF- MalfTLP- ECRC+ UnsupReq+ ACSViol-

- UESvrt: DLP+ SDES+ TLP- FCP+ CmpltTO- CmpltAbrt- UnxCmplt- RxOF+ MalfTLP+ ECRC- UnsupReq- ACSViol-

- CESta:  RxErr- BadTLP- BadDLLP- Rollover- Timeout- AdvNonFatalErr-

- CEMsk:  RxErr- BadTLP- BadDLLP- Rollover- Timeout- AdvNonFatalErr+

- AERCap: First Error Pointer: 00, ECRCGenCap+ ECRCGenEn- ECRCChkCap+ ECRCChkEn-

- MultHdrRecCap- MultHdrRecEn- TLPPfxPres- HdrLogCap-

- HeaderLog: 00000000 00000000 00000000 00000000

- Capabilities: [150 v1] Device Serial Number 9c-bd-6e-56-70-01-28-cf

- Capabilities: [160 v1] Power Budgeting <?>

- Capabilities: [300 v1] Secondary PCI Express

- LnkCtl3: LnkEquIntrruptEn- PerformEqu-

- LaneErrStat: 0

- Capabilities: [400 v1] Vendor Specific Information: ID=0001 Rev=1 Len=010 <?>

- Capabilities: [910 v1] Data Link Feature <?>

- Capabilities: [920 v1] Lane Margining at the Receiver <?>

- Capabilities: [9c0 v1] Physical Layer 16.0 GT/s <?>

- Kernel driver in use: nvme

- Kernel modules: nvme

- [root@bogon ~]#

4、网卡信息与调试

[root@bogon ~]# ip link

1: lo: <LOOPBACK,UP,LOWER_UP> mtu 65536 qdisc noqueue state UNKNOWN mode DEFAULT group default qlen 1000

link/loopback 00:00:00:00:00:00 brd 00:00:00:00:00:00

2: eno1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_codel state UP mode DEFAULT group default qlen 1000

link/ether 2c:da:3f:15:b5:0a brd ff:ff:ff:ff:ff:ff

3: eno2: <NO-CARRIER,BROADCAST,MULTICAST,UP> mtu 1500 qdisc fq_codel state DOWN mode DEFAULT group default qlen 1000

link/ether 2c:da:3f:15:b5:0b brd ff:ff:ff:ff:ff:ff

4: eno3: <NO-CARRIER,BROADCAST,MULTICAST,UP> mtu 1500 qdisc fq_codel state DOWN mode DEFAULT group default qlen 1000

link/ether 2c:da:3f:15:b5:0c brd ff:ff:ff:ff:ff:ff

5: eno4: <NO-CARRIER,BROADCAST,MULTICAST,UP> mtu 1500 qdisc fq_codel state DOWN mode DEFAULT group default qlen 1000

link/ether 2c:da:3f:15:b5:0d brd ff:ff:ff:ff:ff:ff

[root@bogon ~]#

###一张网卡、四个网口、有一个up再用、三个down未在用

并且ethtool eno1的速率为1000MB是1G的网口、为电口、灵活网卡

[root@bogon ~]# ethtool eno1

Settings for eno1:

Supported ports: [ MII ]

Supported link modes:   10baseT/Half 10baseT/Full

100baseT/Half 100baseT/Full

1000baseT/Full

Supported pause frame use: Symmetric Receive-only

Supports auto-negotiation: Yes

Supported FEC modes: Not reported

Advertised link modes:  10baseT/Half 10baseT/Full

100baseT/Half 100baseT/Full

1000baseT/Full

Advertised pause frame use: Symmetric

Advertised auto-negotiation: Yes

Advertised FEC modes: Not reported

Link partner advertised link modes:  10baseT/Half 10baseT/Full

100baseT/Half 100baseT/Full

1000baseT/Full

Link partner advertised pause frame use: Symmetric

Link partner advertised auto-negotiation: Yes

Link partner advertised FEC modes: Not reported

Speed: 1000Mb/s

Duplex: Full

Auto-negotiation: on

Port: MII

PHYAD: 1

Transceiver: internal

Supports Wake-on: d

Wake-on: d

Current message level: 0x00000036 (54)

probe link ifdown ifup

Link detected: yes

[root@bogon ~]#

- 5、PCIe设备与NUMA关系

cat /sys/bus/pci/devices/0000:81:00.0/numa_node查看NVME属于哪个numa

反馈结果为3说明nvme属于numa3

cat /sys/bus/pci/devices/0000:35:00.0/numa_node查看网卡属于哪个numa

- CPU中断（IRQ）与网卡性能

网络中断在哪个cpu处理？

如果网卡在numa2、但是中断跑在cpu0就会跨numa、性能就会下降、

cat /proc/interrupts

查看中断分布eth0-TxRx-0

查看网卡中断cat /proc/interrupts | grep eno1

反馈120:  1000 0 0 0 ...  eno1-TxRx-0说明中断跑在cpu0

高性能服务器会做IRQ affinity

让网卡NUMA = CPU NUMA网络性能最好

进阶部分：（AI服务器/高性能服务器调优必学）

7、PCIe拓扑分析

服务器的结构

CPU

│

Root Port

│

PCIe Switch

│

多个设备

所以服务器会出现多个设备共享带宽

这样看lspci -tv

-+-[0000:80]-

|--81:00.0 NVMe

|--82:00.0 NVMe

|--83:00.0 RAID

这些设备就在同一个Root Port下共享PCIe带宽

如果有4个nvme、都挂在PCIE X16、理论带宽32gb/s

4块nvme就是4*8=32gb/s

刚好跑满、如果在挂一个设备就会带宽竞争性能下降、

如果8个npu、每个需要2gb/s、需求是16gb/s

服务器nvme2个、每个7gb/s、总带宽14gb/s、所以会成为瓶颈、

Nvme numa不一致不等于nvme总带宽不够、

Numa不一致主要影响访问延迟、跨cpu带宽

Nvme-cpu3-cpu2-npu

多了一段cpu-cpu interconnect

Numa不一致会导致带宽下降和延迟增加、

nvme总带宽小于npu需求带宽所以存储带宽不足是瓶颈、而不是numa

但是不会因为npu和nvme不是同一numa

8、CPU亲和性 (CPU Affinity)

服务器程序通常会指定cpu核心、原因是避免cpu迁移

如果程序在cpu间跳来跳去、缓存失效、numa跨结点性能下降

查看cpu绑定：taskset -p $$

pid XXX's current affinity mask说明进程能跑在哪些cpu？？？

指定cpu运行程序：taskset -c 192-200 stress-ng --cpu 4

程序只能跑在numa3、cpu-内存-nvme同一个numa、性能高

Numa绑定运行程序

numactl --cpunodebind=3 --membind=3 stress-ng --cpu 4 --timeout 20

Top查看cpu使用情况

例子：AI服务器gpu在numa1、程序没有绑定numa、推理延迟25ms

Numactl --cpunodebind=1

绑定在numa1：推理延迟18s性能提升30%

挑战题：

8张npu、2nvme'、1rdma nic

如果npu在numa2、nvme在numa3、AI程序的数据流nvme-cpu-npu

程序应该绑定在numa2、

在 AI / HPC / GPU / NPU 场景里通常优先考虑：计算设备优先

CPU NUMA ≈ 加速卡 NUMA

CPU(NUM A2)

│

PCIe

│

NPU

如果绑定NUMA3数据路径

CPU(NUM A3)

│

PCIe

│

NVMe

│

CPU interconnect

│

CPU(NUM A2)

│

PCIe

│

NPU

跨两次numa性能更差

真实流程通常是

NVMe

│

CPU

│

内存

│

NPU

这是关键阶段、因为推理时间远大于数据加载时间

所以优先保证cpu-npu本地numa

NPU → NUMA2

numactl --cpunodebind=2 --membind=2

Nvme数据读取NVMe(NUM A3) → CPU(NUM A2)

虽然也跨numa但是只有一次

而且计算阶段没有跨numa

诊断分析：

NPU 在 NUMA2推理程序应该跑在NUMA2

NPU 挂在 NUMA2 的 PCIe Root Port

程序跑在NUMA2

CPU (NUMA2)

│

PCIe Root Port

│

NPU

延迟最低

带宽最大

如果跑在其他 NUMA

CPU0

│

CPU interconnect

│

CPU1

│

PCIe

│

NPU

多了一段CPU ↔ CPU 互联延迟增加

带宽下降

Day2 Python模块

自动化测试

日志分析

系统监控

批量操作服务器

今天学python＋linux系统信息

读取cpu信息：

创建文件vim cpu_info.py

with open(“/proc/cpuinfo”)as f:

cpuinfo = f.read()

cpu_count = cpuinfo.count(“processor”)

Print(“cpu cores:”,cpu_count)

运行：python3 cpu_info.py

CPU cores: 256

读取内存信息：

创建文件vim mem_info.py

with open(“/proc/meminfo”)as f:

for line in f:

if”MemTotal”in line:

print(line)

运行：python3 mem_info.py

MemTotal: 2111956928 kB

调用 Linux 命令

创建文件vim disk_info.py

import subprocess

result = subprocess.run(["lsblk"], capture_output=True, text=True)

print(result.stdout)

检测 NVMe 数量

import subprocess

result = subprocess.run(["nvme", "list"], capture_output=True, text=True)

count = result.stdout.count("/dev/nvme")

print("NVMe devices:", count)