服务器性能分析、学会判断服务器瓶颈

服务器工程能力

服务器工程师

│

├─Linux系统

│   ├top

│   ├ps

│   ├vmstat

│   └iostat

│

├─CPU架构

│   ├NUMA

│   ├CPU topology

│   └CPU affinity

│

├─PCIe架构

│   ├PCIe Root Port

│   ├PCIe Switch

│   └PCIe Link Speed

│

├─存储系统

│   ├NVMe

│   ├SAS

│   └RAID

│

├─网络系统

│   ├NIC

│   ├RDMA

│   └IRQ

│

└─自动化

├Python

└测试脚本

百日学习计划是为了看到服务器 → 能分析结构

看到性能问题 → 能定位瓶颈

看到设备异常 → 能排查原因

后面的课程会按照以下版块来：

Day1 Linux基础

Day2 服务器硬件识别

Day3 NUMA架构

Day4 PCIe架构

Day5 NVMe存储

Day6 SAS / RAID

Day7 网络 / RDMA

Day8 性能分析

Day9 IRQ与CPU调度

Day10 故障排查

10天其实是 一个小循环。

后面会不断加深。

Day1 Linux基础

Day2 硬件识别

Day3 NUMA

Day4 PCIe

Day5 Storage

Day6 Network

Day7 Performance

Day8 IRQ

Day9 Troubleshooting

Day10 Automation

硬件方向：PCIe / NUMA / NVMe / 服务器整机

AI方向：GPU / RDMA / 分布式训练

系统方向：Linux内核 / 调度 / 系统架构

DAY3

模块1 服务器负载 load average

模块2 CPU使用率 top

模块3 CPU进程分析

模块4 内存分析 free

模块5 内存细节 /proc/meminfo

模块6 IO分析 iostat

模块7 进程资源 pidstat

模块8 系统运行时间 uptime

模块9 CPU中断 /proc/interrupts

模块10 watch实时监控

1、系统整体负载

[root@bogon ~]# uptime

12:42:40 up 22:25,  2 users,  load average: 0.00, 0.00, 0.00

[root@bogon ~]#

三个值分别是

1分钟平均负载

5分钟平均负载

15分钟平均负载

Stress-ng --cpu 256

下面是未跑stress的状态

2、 CPU使用率分析

us 用户进程
sy 内核
id 空闲
wa IO等待

[root@bogon ~]# top

top - 12:32:52 up 22:15,  2 users,  load average: 0.00, 0.00, 0.00

Tasks: 832 total,   2 running, 830 sleeping,   0 stopped,   0 zombie

%Cpu(s):  0.0 us,  0.0 sy,  0.0 ni,100.0 id,  0.0 wa,  0.0 hi,  0.0 si,  0.0 st

MiB Mem : 255968.2 total, 253765.1 free,   3330.2 used,    829.1 buff/cache

MiB Swap:   4096.0 total,   4096.0 free,      0.0 used. 252638.0 avail Mem

PID USER      PR  NI    VIRT    RES    SHR S  %CPU  %MEM     TIME+ COMMAND

50500 root      20   0   27720   5260   3072 R   0.7   0.0   0:00.15 top

48715 root      20   0       0      0      0 I   0.3   0.0   0:00.20 kworker/u160:4-hclge

1 root      20   0  174244  17552   9640 S   0.0   0.0   0:04.15 systemd

2 root      20   0       0      0      0 S   0.0   0.0   0:00.08 kthreadd

3 root       0 -20       0      0      0 I   0.0   0.0   0:00.00 rcu_gp

4 root       0 -20       0      0      0 I   0.0   0.0   0:00.00 rcu_par_gp

6 root       0 -20       0      0      0 I   0.0   0.0   0:00.00 kworker/0:0H-events_highpri

8 root       0 -20       0      0      0 I   0.0   0.0   0:00.00 mm_percpu_wq

9 root      20   0       0      0      0 S   0.0   0.0   0:00.00 rcu_tasks_rude_

10 root      20   0       0      0      0 S   0.0   0.0   0:00.00 rcu_tasks_trace

11 root      20   0       0      0      0 S   0.0   0.0   0:00.00 ksoftirqd/0

12 root      20   0       0      0      0 I   0.0   0.0   0:04.67 rcu_sched

13 root      rt   0       0      0      0 S   0.0   0.0   0:00.00 migration/0

14 root      20   0       0      0      0 S   0.0   0.0   0:00.00 cpuhp/0

15 root      20   0       0      0      0 S   0.0   0.0   0:00.00 cpuhp/1

16 root      rt   0       0      0      0 S   0.0   0.0   0:00.00 migration/1

17 root      20   0       0      0      0 S   0.0   0.0   0:00.00 ksoftirqd/1

18 root      20   0       0      0      0 I   0.0   0.0   0:00.00 kworker/1:0-events

19 root       0 -20       0      0      0 I   0.0   0.0   0:00.00 kworker/1:0H-events_highpri

20 root      20   0       0      0      0 S   0.0   0.0   0:00.00 cpuhp/2

21 root      rt   0       0      0      0 S   0.0   0.0   0:00.02 migration/2

22 root      20   0       0      0      0 S   0.0   0.0   0:00.03 ksoftirqd/2

24 root       0 -20       0      0      0 I   0.0   0.0   0:00.00 kworker/2:0H-events_highpri

25 root      20   0       0      0      0 S   0.0   0.0   0:00.00 cpuhp/3

26 root      rt   0       0      0      0 S   0.0   0.0   0:00.01 migration/3

27 root      20   0       0      0      0 S   0.0   0.0   0:00.02 ksoftirqd/3

29 root       0 -20       0      0      0 I   0.0   0.0   0:00.00 kworker/3:0H-events_highpri

30 root      20   0       0      0      0 S   0.0   0.0   0:00.00 cpuhp/4

31 root      rt   0       0      0      0 S   0.0   0.0   0:00.01 migration/4

32 root      20   0       0      0      0 S   0.0   0.0   0:00.04 ksoftirqd/4

34 root       0 -20       0      0      0 I   0.0   0.0   0:00.00 kworker/4:0H-events_highpri

35 root      20   0       0      0      0 S   0.0   0.0   0:00.00 cpuhp/5

36 root      rt   0       0      0      0 S   0.0   0.0   0:00.01 migration/5

37 root      20   0       0      0      0 S   0.0   0.0   0:00.04 ksoftirqd/5

39 root       0 -20       0      0      0 I   0.0   0.0   0:00.00 kworker/5:0H-events_highpri

40 root      20   0       0      0      0 S   0.0   0.0   0:00.00 cpuhp/6

41 root      rt   0       0      0      0 S   0.0   0.0   0:00.01 migration/6

42 root      20   0       0      0      0 S   0.0   0.0   0:00.04 ksoftirqd/6

44 root       0 -20       0      0      0 I   0.0   0.0   0:00.00 kworker/6:0H-events_highpri

45 root      20   0       0      0      0 S   0.0   0.0   0:00.00 cpuhp/7

46 root      rt   0       0      0      0 S   0.0   0.0   0:00.01 migration/7

47 root      20   0       0      0      0 S   0.0   0.0   0:00.04 ksoftirqd/7

按h查看help

Help for Interactive Commands - procps-ng 4.0.2

Window 1:Def: Cumulative mode Off.  System: Delay 3.0 secs; Secure mode Off.

Z,B,E,e   Global: 'Z' colors; 'B' bold; 'E'/'e' summary/task memory scale

l,t,m,I   Toggle: 'l' load avg; 't' task/cpu; 'm' memory; 'I' Irix mode

0,1,2,3,4 Toggle: '0' zeros; '1/2/3' cpu/numa views; '4' cpus two abreast

f,X       Fields: 'f' add/remove/order/sort; 'X' increase fixed-width fields

L,&,<,> . Locate: 'L'/'&' find/again; Move sort column: '<'/'>' left/right

R,H,J,C . Toggle: 'R' Sort; 'H' Threads; 'J' Num justify; 'C' Coordinates

c,i,S,j . Toggle: 'c' Cmd name/line; 'i' Idle; 'S' Time; 'j' Str justify

x,y     . Toggle highlights: 'x' sort field; 'y' running tasks

z,b     . Toggle: 'z' color/mono; 'b' bold/reverse (only if 'x' or 'y')

u,U,o,O . Filter by: 'u'/'U' effective/any user; 'o'/'O' other criteria

n,#,^O  . Set: 'n'/'#' max tasks displayed; Show: Ctrl+'O' other filter(s)

V,v,F   . Toggle: 'V' forest view; 'v' hide/show children; 'F' keep focused

d,k,r,^R 'd' set delay; 'k' kill; 'r' renice; Ctrl+'R' renice autogroup

^G,K,N,U  View: ctl groups ^G; cmdline ^K; environment ^N; supp groups ^U

W,Y,!,^E  Write cfg 'W'; Inspect 'Y'; Combine Cpus '!'; Scale time ^E'

q         Quit

( commands shown with '.' require a visible task display window )

Press 'h' or '?' for help with Windows,

Type 'q' or <Esc> to continue

3、CPU进程分析

鲲鹏920

按p进不去？？

4、内存使用分析

[root@bogon ~]# free -h

total        used        free      shared      buff/cache   available

Mem:          249Gi       3.2Gi       247Gi       9.9Mi       834Mi       246Gi

Swap:          4.0Gi          0B       4.0Gi

[root@bogon ~]#

##available 很低说明内存不够用

5、模块5：查看内存细节：关注

MemTotal

MemFree

MemAvailable

cat /proc/meminfo | head

6、模块6 IO分析 iostat

IO性能分析

[root@bogon ~]# iostat -x 1（-x 1每一秒输出一次）

Linux 5.10.0-216.0.0.115.oe2203sp4.aarch64 (bogon)      2026年03月11日  _aarch64_       (80 CPU)

avg-cpu:  %user   %nice %system %iowait  %steal   %idle

0.01    0.00    0.01    0.00    0.00   99.99

Device            r/s     rkB/s   rrqm/s  %rrqm r_await rareq-sz     w/s     wkB/s   wrqm/s  %wrqm w_await wareq-sz     d/s     dkB/s   drqm/s  %drqm d_await dareq-sz     f/s f_await  aqu-sz  %util

dm-0             0.13      3.06     0.00   0.00    0.23    24.26    1.03     21.02     0.00   0.00    0.02    20.43    0.00      0.00     0.00   0.00    0.00     0.00    0.00    0.00    0.00   0.09

dm-1             0.00      0.03     0.00   0.00    0.30    70.40    0.00      0.00     0.00   0.00    0.00     0.00    0.00      0.00     0.00   0.00    0.00     0.00    0.00    0.00    0.00   0.00

dm-2             0.00      0.05     0.00   0.00    0.10    27.41    0.00      0.01     0.00   0.00    0.00     4.00    0.00      0.00     0.00   0.00    0.00     0.00    0.00    0.00    0.00   0.00

sda              0.11      3.90     0.05  33.10    0.41    35.81    0.68     21.03     0.46  40.69    0.24    31.12    0.00      0.00     0.00   0.00    0.00     0.00    0.00    0.00    0.00   0.00

avg-cpu:  %user   %nice %system %iowait  %steal   %idle

0.01    0.00    0.01    0.00    0.00   99.98

Device            r/s     rkB/s   rrqm/s  %rrqm r_await rareq-sz     w/s     wkB/s   wrqm/s  %wrqm w_await wareq-sz     d/s     dkB/s   drqm/s  %drqm d_await dareq-sz     f/s f_await  aqu-sz  %util

dm-0             0.00      0.00     0.00   0.00    0.00     0.00    0.00      0.00     0.00   0.00    0.00     0.00    0.00      0.00     0.00   0.00    0.00     0.00    0.00    0.00    0.00   0.00

dm-1             0.00      0.00     0.00   0.00    0.00     0.00    0.00      0.00     0.00   0.00    0.00     0.00    0.00      0.00     0.00   0.00    0.00     0.00    0.00    0.00    0.00   0.00

dm-2             0.00      0.00     0.00   0.00    0.00     0.00    0.00      0.00     0.00   0.00    0.00     0.00    0.00      0.00     0.00   0.00    0.00     0.00    0.00    0.00    0.00   0.00

sda              0.00      0.00     0.00   0.00    0.00     0.00    0.00      0.00     0.00   0.00    0.00     0.00    0.00      0.00     0.00   0.00    0.00     0.00    0.00    0.00    0.00   0.00

%util 100说明磁盘已经跑满

先下载yum install sysstat

7、模块进程资源 pidstat

系统提示无Pidstat命令

- 模块8 系统运行时间 uptime

Uptime

模块9 CPU中断 /proc/interrupts

模块10 watch实时监控

Day3 Python模块（服务器工程版本）

用python读取服务器信息

import subprocess

result = subprocess.run(["lscpu"], capture_output=True, text=True)

print(result.stdout)

功能等同于lscpu

读取cpu数

import subprocess

result = subprocess.run(["lscpu"], capture_output=True, text=True)

for line in result.stdout.split("\n"):

if "CPU(s):" in line:

print(line)

等同于：

lscpu | grep -i cpu：

Cpu: 256

读取NVME数量

vim nvme_check.py

import subprocess

result = subprocess.run(["nvme", "list"], capture_output=True, text=True)

count = result.stdout.count("/dev/nvme")

print("NVMe devices:", count)

可以看到nvme数量是2

同意nvme list也可以看到有几个nmve

综合脚本

import subprocess

print("=== CPU ===")

print(subprocess.run(["lscpu"], capture_output=True, text=True).stdout)

print("=== Memory ===")

print(subprocess.run(["free","-h"], capture_output=True, text=True).stdout)

print("=== NVMe ===")

print(subprocess.run(["nvme","list"], capture_output=True, text=True).stdout)