Day4：PCIe 架构与服务器设备拓扑（完整版）

1 PCIe基础概念

2 PCIe lane

3 PCIe Gen版本

4 PCIe带宽计算

5 PCIe设备识别

6 PCIe链路状态

7 PCIe降速判断

8 PCIe拓扑

9 PCIe Root Port

10 PCIe Switch

11 设备NUMA关系

12 Python自动化检测PCIe

我们把 Day4扩展成完整训练版：12个模块。
这样一天基本就是 3~5小时工程训练量。

Day4：PCIe 架构与服务器设备拓扑（完整版）

今天 12个模块：彻底看懂服务器PCIe结构

1 PCIe基础概念

服务器里的很多设备其实都通过 PCIe总线连接到CPU。

NVMe

NIC

GPU / NPU

RAID

USB

SATA

结构是

CPU

│

PCIe Root Port

│

PCIe Switch

│

设备

lspci第一件事！！会看到

Non-Volatile memory controller（nvme）

Ethernet controller网卡

RAID controller raid卡

Processing accelerators

实验一：查看所有PCIe设备

[root@localhost ~]# lspci

01:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

01:01.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

01:02.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

01:03.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

02:00.0 VGA compatible controller: Huawei Technologies Co., Ltd. Hi171x Series [iBMC Intelligent Management system chip w/VGA support] (rev 01)

03:00.0 Signal processing controller: Huawei Technologies Co., Ltd. iBMA Virtual Network Adapter (rev 01)

07:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

08:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

0b:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

0c:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

2e:00.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

2e:01.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

2e:08.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

2e:09.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

2e:0b.0 System peripheral: Huawei Technologies Co., Ltd. HiSilicon Embedded DMA Engine (rev 30)

2f:00.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

2f:01.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

2f:08.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

2f:09.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

2f:0b.0 System peripheral: Huawei Technologies Co., Ltd. HiSilicon Embedded DMA Engine (rev 30)

30:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

30:01.0 USB controller: Huawei Technologies Co., Ltd. Device a23c (rev 30)

30:03.0 USB controller: Huawei Technologies Co., Ltd. Device a23d (rev 30)

30:04.0 Serial Attached SCSI controller: Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA (rev 30)

30:05.0 Memory controller: Huawei Technologies Co., Ltd. Device a12f (rev 30)

32:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

32:01.0 USB controller: Huawei Technologies Co., Ltd. Device a23c (rev 30)

32:03.0 USB controller: Huawei Technologies Co., Ltd. Device a23d (rev 30)

32:04.0 Serial Attached SCSI controller: Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA (rev 30)

34:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

34:01.0 System peripheral: Huawei Technologies Co., Ltd. Device a22b (rev 30)

35:00.0 Ethernet controller: Huawei Technologies Co., Ltd. HNS GE/10GE/25GE/50GE RDMA Network Controller (rev 30)

35:00.1 Ethernet controller: Huawei Technologies Co., Ltd. HNS GE/10GE/25GE/50GE RDMA Network Controller (rev 30)

35:00.2 Ethernet controller: Huawei Technologies Co., Ltd. HNS GE/10GE/25GE/50GE RDMA Network Controller (rev 30)

35:00.3 Ethernet controller: Huawei Technologies Co., Ltd. HNS GE/10GE/25GE/50GE RDMA Network Controller (rev 30)

36:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

38:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

38:01.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

38:05.0 SATA controller: Huawei Technologies Co., Ltd. HiSilicon AHCI HBA (rev 30)

38:08.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

38:09.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

3c:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

3c:01.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

3c:08.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

3c:09.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

41:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

42:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

45:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

46:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

6e:00.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

6e:01.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

6e:08.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

6e:09.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

6e:0b.0 System peripheral: Huawei Technologies Co., Ltd. HiSilicon Embedded DMA Engine (rev 30)

6f:00.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

6f:01.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

6f:08.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

6f:09.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

6f:0b.0 System peripheral: Huawei Technologies Co., Ltd. HiSilicon Embedded DMA Engine (rev 30)

70:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

70:01.0 USB controller: Huawei Technologies Co., Ltd. Device a23c (rev 30)

70:03.0 USB controller: Huawei Technologies Co., Ltd. Device a23d (rev 30)

70:04.0 Serial Attached SCSI controller: Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA (rev 30)

72:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

72:01.0 USB controller: Huawei Technologies Co., Ltd. Device a23c (rev 30)

72:03.0 USB controller: Huawei Technologies Co., Ltd. Device a23d (rev 30)

72:04.0 Serial Attached SCSI controller: Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA (rev 30)

74:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

76:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

78:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

78:01.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

78:08.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

78:09.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

7c:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

7c:01.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

7c:08.0 System peripheral: Huawei Technologies Co., Ltd. Device a12d (rev 30)

7c:09.0 System peripheral: Huawei Technologies Co., Ltd. Device a12e (rev 30)

80:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

80:02.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

80:04.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

81:00.0 Non-Volatile memory controller: DERA Storage Device 1515

82:00.0 Non-Volatile memory controller: DERA Storage Device 1515

83:00.0 RAID bus controller: Broadcom / LSI MegaRAID 12GSAS/PCIe Secure SAS39xx

95:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

96:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

aa:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

ab:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

c0:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

c1:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)（NPU/GPU）

d5:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

d6:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

[root@localhost ~]#

2 PCIe lane

PCIe lanePCIe是 多通道通信。

常见宽度

x1

x4

x8

x16

实验二：查看NVMe PCIe链路

NVMe → x4

GPU → x16

NIC → x8

[root@localhost ~]# lspci | grep -i nvme

[root@localhost ~]# lspci | grep -i non

81:00.0 Non-Volatile memory controller: DERA Storage Device 1515

82:00.0 Non-Volatile memory controller: DERA Storage Device 1515

[root@localhost ~]# lspci | grep -i 81:00.0

81:00.0 Non-Volatile memory controller: DERA Storage Device 1515

[root@localhost ~]#

[root@localhost ~]# lspci -vv -s 81:00.0

81:00.0 Non-Volatile memory controller: DERA Storage Device 1515 (prog-if 02 [NVM Express])

Subsystem: DERA Storage Device 7105

Physical Slot: 67

Control: I/O+ Mem+ BusMaster+ SpecCycle- MemWINV- VGASnoop- ParErr+ Stepping- SERR+ FastB2B- DisINTx+

Status: Cap+ 66MHz- UDF- FastB2B- ParErr- DEVSEL=fast >TAbort- <TAbort- <MAbort- >SERR- <PERR- INTx-

Latency: 0, Cache Line Size: 32 bytes

Interrupt: pin A routed to IRQ 28

NUMA node: 3

Region 0: Memory at a0320000 (64-bit, non-prefetchable) [size=32K]

Expansion ROM at a0300000 [disabled] [size=128K]

Capabilities: [80] Power Management version 3

Flags: PMEClk- DSI- D1- D2- AuxCurrent=0mA PME(D0-,D1-,D2-,D3hot-,D3cold-)

Status: D0 NoSoftRst+ PME-Enable- DSel=0 DScale=0 PME-

Capabilities: [90] MSI: Enable- Count=1/1 Maskable+ 64bit+

Address: 0000000000000000  Data: 0000

Masking: 00000000  Pending: 00000000

Capabilities: [b0] MSI-X: Enable+ Count=257 Masked-

Vector table: BAR=0 offset=00004000

PBA: BAR=0 offset=00006000

Capabilities: [c0] Express (v2) Endpoint, MSI 00

DevCap: MaxPayload 512 bytes, PhantFunc 0, Latency L0s <1us, L1 <1us

ExtTag- AttnBtn- AttnInd- PwrInd- RBE+ FLReset+ SlotPowerLimit 0W

DevCtl: CorrErr+ NonFatalErr+ FatalErr+ UnsupReq+

RlxdOrd+ ExtTag- PhantFunc- AuxPwr- NoSnoop+ FLReset-

MaxPayload 256 bytes, MaxReadReq 512 bytes

DevSta: CorrErr- NonFatalErr- FatalErr- UnsupReq- AuxPwr- TransPend-

LnkCap: Port #0, Speed 16GT/s, Width x4, ASPM not supported

ClockPM- Surprise- LLActRep- BwNot- ASPMOptComp+

LnkCtl: ASPM Disabled; RCB 128 bytes, Disabled- CommClk-

ExtSynch- ClockPM- AutWidDis- BWInt- AutBWInt-

LnkSta: Speed 16GT/s, Width x4

TrErr- Train- SlotClk- DLActive- BWMgmt- ABWMgmt-

DevCap2: Completion Timeout: Range B, TimeoutDis+ NROPrPrP- LTR+

10BitTagComp+ 10BitTagReq- OBFF Via message, ExtFmt+ EETLPPrefix+, MaxEETLPPrefixes 2

EmergencyPowerReduction Not Supported, EmergencyPowerReductionInit-

FRS- TPHComp+ ExtTPHComp+

AtomicOpsCap: 32bit- 64bit- 128bitCAS-

DevCtl2: Completion Timeout: 65ms to 210ms, TimeoutDis- LTR- 10BitTagReq- OBFF Disabled,

AtomicOpsCtl: ReqEn-

LnkCap2: Supported Link Speeds: 2.5-16GT/s, Crosslink- Retimer+ 2Retimers+ DRS-

LnkCtl2: Target Link Speed: 16GT/s, EnterCompliance- SpeedDis-

Transmit Margin: Normal Operating Range, EnterModifiedCompliance- ComplianceSOS-

Compliance Preset/De-emphasis: -6dB de-emphasis, 0dB preshoot

LnkSta2: Current De-emphasis Level: -6dB, EqualizationComplete+ EqualizationPhase1+

EqualizationPhase2+ EqualizationPhase3+ LinkEqualizationRequest-

Retimer- 2Retimers- CrosslinkRes: unsupported

Capabilities: [100 v2] Advanced Error Reporting

UESta:  DLP- SDES- TLP- FCP- CmpltTO- CmpltAbrt- UnxCmplt- RxOF- MalfTLP- ECRC- UnsupReq- ACSViol-

UEMsk:  DLP- SDES- TLP- FCP- CmpltTO- CmpltAbrt- UnxCmplt- RxOF- MalfTLP- ECRC+ UnsupReq+ ACSViol-

UESvrt: DLP+ SDES+ TLP- FCP+ CmpltTO- CmpltAbrt- UnxCmplt- RxOF+ MalfTLP+ ECRC- UnsupReq- ACSViol-

CESta:  RxErr- BadTLP- BadDLLP- Rollover- Timeout- AdvNonFatalErr-

CEMsk:  RxErr- BadTLP- BadDLLP- Rollover- Timeout- AdvNonFatalErr+

AERCap: First Error Pointer: 00, ECRCGenCap+ ECRCGenEn- ECRCChkCap+ ECRCChkEn-

MultHdrRecCap- MultHdrRecEn- TLPPfxPres- HdrLogCap-

HeaderLog: 00000000 00000000 00000000 00000000

Capabilities: [150 v1] Device Serial Number 9c-bd-6e-56-70-01-28-cf

Capabilities: [160 v1] Power Budgeting <?>

Capabilities: [300 v1] Secondary PCI Express

LnkCtl3: LnkEquIntrruptEn- PerformEqu-

LaneErrStat: 0

Capabilities: [400 v1] Vendor Specific Information: ID=0001 Rev=1 Len=010 <?>

Capabilities: [910 v1] Data Link Feature <?>

Capabilities: [920 v1] Lane Margining at the Receiver <?>

Capabilities: [9c0 v1] Physical Layer 16.0 GT/s <?>

Kernel driver in use: nvme

Kernel modules: nvme

[root@localhost ~]#

重点看

LnkCap: Speed 16GT/s Width x4

LnkSta: Speed 16GT/s Width x4

16GT/s → PCIe Gen4

x4 → 4条lane带宽约等于≈8GB/s

3 PCIe Gen版本

服务器设备不是直接挂在CPU上。

通常结构是

CPU

│

Root Port

│

Switch

│

多个设备

查看拓扑命令

lspci -tv

实验3：查看完整PCIe拓扑

重点观察

Root Port

Switch

NVMe

NIC

NPU

[root@localhost ~]# lspci -tv

-+-[0000:00]-

+-[0000:01]-+-00.0-[02]----00.0  Huawei Technologies Co., Ltd. Hi171x Series [iBMC Intelligent Management system chip w/VGA support]

|           +-01.0-[03]----00.0  Huawei Technologies Co., Ltd. iBMA Virtual Network Adapter

|           +-02.0-[04]--

|           \-03.0-[05]--

+-[0000:07]---00.0-[08]----00.0  Huawei Technologies Co., Ltd. Device d802

+-[0000:0b]---00.0-[0c]----00.0  Huawei Technologies Co., Ltd. Device d802

+-[0000:2e]-+-00.0  Huawei Technologies Co., Ltd. Device a12d

|           +-01.0  Huawei Technologies Co., Ltd. Device a12e

|           +-08.0  Huawei Technologies Co., Ltd. Device a12d

|           +-09.0  Huawei Technologies Co., Ltd. Device a12e

|           \-0b.0  Huawei Technologies Co., Ltd. HiSilicon Embedded DMA Engine

+-[0000:2f]-+-00.0  Huawei Technologies Co., Ltd. Device a12d

|           +-01.0  Huawei Technologies Co., Ltd. Device a12e

|           +-08.0  Huawei Technologies Co., Ltd. Device a12d

|           +-09.0  Huawei Technologies Co., Ltd. Device a12e

|           \-0b.0  Huawei Technologies Co., Ltd. HiSilicon Embedded DMA Engine

+-[0000:30]-+-00.0-[31]--

|           +-01.0  Huawei Technologies Co., Ltd. Device a23c

|           +-03.0  Huawei Technologies Co., Ltd. Device a23d

|           +-04.0  Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA

|           \-05.0  Huawei Technologies Co., Ltd. Device a12f

+-[0000:32]-+-00.0-[33]--

|           +-01.0  Huawei Technologies Co., Ltd. Device a23c

|           +-03.0  Huawei Technologies Co., Ltd. Device a23d

|           \-04.0  Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA

+-[0000:34]-+-00.0-[35]--+-00.0  Huawei Technologies Co., Ltd. HNS GE/10GE/25GE/50GE RDMA Network Controller

|           |            +-00.1  Huawei Technologies Co., Ltd. HNS GE/10GE/25GE/50GE RDMA Network Controller

|           |            +-00.2  Huawei Technologies Co., Ltd. HNS GE/10GE/25GE/50GE RDMA Network Controller

|           |            \-00.3  Huawei Technologies Co., Ltd. HNS GE/10GE/25GE/50GE RDMA Network Controller

|           \-01.0  Huawei Technologies Co., Ltd. Device a22b

一个RDMA：35:00.0

35:00.1

35:00.2

35:00.3

一张网卡4个网口

Root Port

└ NIC

+-[0000:36]---00.0-[37]--

+-[0000:38]-+-00.0-[39]--

|           +-01.0-[3a]--

|           +-05.0  Huawei Technologies Co., Ltd. HiSilicon AHCI HBA

|           +-08.0  Huawei Technologies Co., Ltd. Device a12d

|           \-09.0  Huawei Technologies Co., Ltd. Device a12e

+-[0000:3c]-+-00.0-[3d]--

|           +-01.0-[3e]--

|           +-08.0  Huawei Technologies Co., Ltd. Device a12d

|           \-09.0  Huawei Technologies Co., Ltd. Device a12e

+-[0000:41]---00.0-[42]----00.0  Huawei Technologies Co., Ltd. Device d802

+-[0000:45]---00.0-[46]----00.0  Huawei Technologies Co., Ltd. Device d802

+-[0000:6e]-+-00.0  Huawei Technologies Co., Ltd. Device a12d

|           +-01.0  Huawei Technologies Co., Ltd. Device a12e

|           +-08.0  Huawei Technologies Co., Ltd. Device a12d

|           +-09.0  Huawei Technologies Co., Ltd. Device a12e

|           \-0b.0  Huawei Technologies Co., Ltd. HiSilicon Embedded DMA Engine

+-[0000:6f]-+-00.0  Huawei Technologies Co., Ltd. Device a12d

|           +-01.0  Huawei Technologies Co., Ltd. Device a12e

|           +-08.0  Huawei Technologies Co., Ltd. Device a12d

|           +-09.0  Huawei Technologies Co., Ltd. Device a12e

|           \-0b.0  Huawei Technologies Co., Ltd. HiSilicon Embedded DMA Engine

+-[0000:70]-+-00.0-[71]--

|           +-01.0  Huawei Technologies Co., Ltd. Device a23c

|           +-03.0  Huawei Technologies Co., Ltd. Device a23d

|           \-04.0  Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA

+-[0000:72]-+-00.0-[73]--

|           +-01.0  Huawei Technologies Co., Ltd. Device a23c

|           +-03.0  Huawei Technologies Co., Ltd. Device a23d

|           \-04.0  Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA

+-[0000:74]---00.0-[75]--

+-[0000:76]---00.0-[77]--

+-[0000:78]-+-00.0-[79]--

|           +-01.0-[7a]--

|           +-08.0  Huawei Technologies Co., Ltd. Device a12d

|           \-09.0  Huawei Technologies Co., Ltd. Device a12e

+-[0000:7c]-+-00.0-[7d]--

|           +-01.0-[7e]--

|           +-08.0  Huawei Technologies Co., Ltd. Device a12d

|           \-09.0  Huawei Technologies Co., Ltd. Device a12e

+-[0000:80]-+-00.0-[81]----00.0  DERA Storage Device 1515

|           +-02.0-[82]----00.0  DERA Storage Device 1515

|           \-04.0-[83]----00.0  Broadcom / LSI MegaRAID 12GSAS/PCIe

## +-[0000:80]-+-00.0-[81]----00.0  DERA Storage Device 1515

|           +-02.0-[82]----00.0  DERA Storage Device 1515

|           \-04.0-[83]----00.0  Broadcom / LSI MegaRAID 12GSAS/PCIe

说明三个设备挂在同一个pcie root port上

结构是CPU

└ RootPort (80)

├ NVMe

├ NVMe

└ RAID

共享 PCIe 带宽

Secure SAS39xx

+-[0000:95]---00.0-[96]----00.0  Huawei Technologies Co., Ltd. Device d802

+-[0000:aa]---00.0-[ab]----00.0  Huawei Technologies Co., Ltd. Device d802

+-[0000:c0]---00.0-[c1]----00.0  Huawei Technologies Co., Ltd. Device d802

\-[0000:d5]---00.0-[d6]----00.0  Huawei Technologies Co., Ltd. Device d802 #atlasnpu

[root@localhost ~]#

模块4：PCIe设备通常绑定在 某个NUMA节点。

[root@localhost ~]# cat /sys/bus/pci/devices/0000:81:00.0/numa_node

3

[root@localhost ~]#

说明NVMe在NUMA3

这就是cpu要绑定numa

实验4：查看NPU NUMA

lspci | grep -i accelerator

[root@localhost ~]# lspci | grep -i accelerator

08:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

0c:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

42:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

46:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

96:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

ab:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

c1:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

d6:00.0 Processing accelerators: Huawei Technologies Co., Ltd. Device d802 (rev 20)

[root@localhost ~]#

8张npu、每张一个pcie root port

CPU

└ RootPort

└ NPU

[root@localhost ~]# cat /sys/bus/pci/devices/0000:08:00.0/numa_node

1

[root@localhost ~]#

说明0000:08:00.0绑定在numa1

模块5：模块5：PCIe降速

LnkCap: 16GT/s

LnkSta: 8GT/s

设备本来支持Gen4

但现在跑Gen3

BIOS限制

兼容性

信号问题

4 PCIe带宽计算

5 PCIe设备识别

6 PCIe链路状态

7 PCIe降速判断

8 PCIe拓扑

CPU0

├ RootPort

│   └ NPU (08:00.0)

├ RootPort

│   └ NPU (0c:00.0)

├ RootPort

│   └ NIC

└ RootPort

├ NVMe

├ NVMe

└ RAID

CPU1

├ RootPort

│   └ NPU (42:00.0)

├ RootPort

│   └ NPU (46:00.0)

├ RootPort

│   └ NPU (96:00.0)

├ RootPort

│   └ NPU (ab:00.0)

├ RootPort

│   └ NPU (c1:00.0)

└ RootPort

└ NPU (d6:00.0)

服务器一般这样设计

CPU0 → Storage / Network

CPU1 → Compute (GPU/NPU)

NVMe → 数据读取

NPU → AI计算

NVMe

↓

CPU

↓

NPU

如果 NVMe 和 NPU 在 不同 CPU：

就会出现：

跨 CPU
跨 NUMA

带宽会下降。

[root@localhost ~]# ##nvme

[root@localhost ~]# cat /sys/bus/pci/devices/0000:81:00.0/numa_node

3

[root@localhost ~]# cat /sys/bus/pci/devices/0000:82:00.0/numa_node

3

[root@localhost ~]# ##NPU

[root@localhost ~]# cat /sys/bus/pci/devices/0000:08:00.0/numa_node

1

[root@localhost ~]# cat /sys/bus/pci/devices/0000:0c:00.0/numa_node

1

[root@localhost ~]# cat /sys/bus/pci/devices/0000:42:00.0/numa_node

0

[root@localhost ~]# cat /sys/bus/pci/devices/0000:46:00.0/numa_node

0

[root@localhost ~]# cat /sys/bus/pci/devices/0000:96:00.0/numa_node

3

[root@localhost ~]# cat /sys/bus/pci/devices/0000:ab:00.0/numa_node

3

[root@localhost ~]# cat /sys/bus/pci/devices/0000:c1:00.0/numa_node

2

[root@localhost ~]# cat /sys/bus/pci/devices/0000:d6:00.0/numa_node

2

[root@localhost ~]# ##nic

[root@localhost ~]# cat /sys/bus/pci/devices/0000:35:00.0/numa_node

1

[root@localhost ~]# cat /sys/bus/pci/devices/0000:35:00.1/numa_node

1

[root@localhost ~]# cat /sys/bus/pci/devices/0000:35:00.2/numa_node

1

[root@localhost ~]# cat /sys/bus/pci/devices/0000:35:00.3/numa_node

1

[root@localhost ~]#

[root@localhost ~]# lscpu | grep NUMA

NUMA 节点：                         4

NUMA 节点0 CPU：                    0-63

NUMA 节点1 CPU：                    64-127

NUMA 节点2 CPU：                    128-191

NUMA 节点3 CPU：                    192-255

[root@localhost ~]#

二、你这台服务器的真实结构

根据你所有信息，可以整理成：

Socket0
 NUMA0
 NUMA1

Socket1
 NUMA2
 NUMA3

CPU：

256 CPU
每NUMA 64

设备分布：

NUMA0

NPU
NPU

NUMA1

NPU
NPU
NIC

NUMA2

NPU
NPU

NUMA3

NVMe
NVMe
RAID
NPU
NPU

U.2 只是物理接口

通信仍然是 PCIe

U.2 的nvme只是接口形式不同

本质还是pcie通信

U.2 NVMe 一般是：PCIe Gen4 x4理论带宽≈ 8GB/s

CPU

│

PCIe Root Port

│

PCIe switch / retimer

│

U.2 backplane支持热插拔 支持多盘位（8盘

16盘

24盘）

│

U.2 NVMe

服务器存储架构

根据所有信息，机器的存储大概是：

CPU
 └ RootPort
     ├ NVMe (U.2)
     ├ NVMe (U.2)
     └ RAID (SAS)
          └ SAS SSD × 8

也就是：

NVMe + SAS 混合架构

六、一个非常关键的服务器工程问题

假设 AI任务：

NVMe → CPU → NPU

而你机器：

NVMe → NUMA3
NPU → NUMA1

数据路径会变成：

NVMe
 ↓
CPU1
 ↓
跨Socket
 ↓
CPU0
 ↓
NPU

也就是：

跨NUMA
跨CPU

这就是：

AI服务器性能优化

的核心问题。

实验1：Python执行 lspci

创建脚本：

vim check_pcie.py

代码：

import subprocess

result = subprocess.run(["lspci"], capture_output=True, text=True)

print(result.stdout)

运行：

python3 check_pcie.py

作用：

Python执行Linux命令

这是自动化测试的基础。

实验2：统计 NVMe数量

修改脚本：

import subprocess

result = subprocess.run(["lspci"], capture_output=True, text=True)

nvme_count = result.stdout.count("Non-Volatile")

print("NVMe数量:", nvme_count)

运行：

python3 check_pcie.py

输出类似：

NVMe数量: 2

实验3：统计 NPU数量

修改脚本：

import subprocess

result = subprocess.run(["lspci"], capture_output=True, text=True)

npu_count = result.stdout.count("Processing accelerators")

print("NPU数量:", npu_count)

你服务器应该输出：

NPU数量: 8

实验4：统计 NIC数量

再增加：

nic_count = result.stdout.count("Ethernet")

print("NIC数量:", nic_count)

完整脚本：

import subprocess

result = subprocess.run(["lspci"], capture_output=True, text=True)

nvme = result.stdout.count("Non-Volatile")
npu = result.stdout.count("Processing accelerators")
nic = result.stdout.count("Ethernet")

print("NVMe:", nvme)
print("NPU:", npu)
print("NIC:", nic)

运行：

python3 check_pcie.py

实验5：服务器自动化检测脚本（工程版）

再进阶一点：

import subprocess

result = subprocess.run(["lspci"], capture_output=True, text=True)

nvme = result.stdout.count("Non-Volatile")
npu = result.stdout.count("Processing accelerators")
nic = result.stdout.count("Ethernet")

if nvme != 2:
    print("NVMe数量异常")

if npu != 8:
    print("NPU数量异常")

if nic < 1:
    print("NIC异常")

print("检查完成")

这其实就是：

服务器Bringup检测脚本

很多厂商都会写这种。

模块12：Python自动化检测 PCIe设备

第一步：确认服务器结构

你的服务器：

2 Socket
256 CPU
4 NUMA

基本结构其实是：

CPU0
 ├ NUMA0
 └ NUMA1

CPU1
 ├ NUMA2
 └ NUMA3

每个 CPU 内部都有很多：

PCIe Root Port

第二步：识别 Root Port

lspci 输出里有很多：

PCI bridge: HiSilicon PCIe Root Port with Gen4

[root@localhost ~]# lspci | grep -i bridge

01:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

01:01.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

01:02.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

01:03.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

07:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

0b:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

30:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

32:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

34:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

36:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

38:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

38:01.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

3c:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

3c:01.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

41:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

45:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

70:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

72:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

74:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

76:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

78:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

78:01.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

7c:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

7c:01.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCI-PCI Bridge (rev 30)

80:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

80:02.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

80:04.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

95:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

aa:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

c0:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

d5:00.0 PCI bridge: Huawei Technologies Co., Ltd. HiSilicon PCIe Root Port with Gen4 (rev 30)

[root@localhost ~]#

例如：

01:00.0 PCI bridge
01:01.0 PCI bridge
01:02.0 PCI bridge
01:03.0 PCI bridge

这些就是：

CPU → PCIe Root Port

Root Port 的作用是：

CPU 和 PCIe 设备之间的入口

9 PCIe Root Port

10 PCIe Switch

11 设备NUMA关系

[root@localhost ~]# cat /sys/bus/pci/devices/0000:81:00.0/numa_node

3

[root@localhost ~]#

说明NVMe在NUMA3

12 Python自动化检测PCIe

模块1：PCIe是什么

PCIe是服务器 高速设备总线。

典型结构：

CPU

│

PCIe Root Port

│

PCIe Switch

│

设备

设备包括：

NVMe

NIC

GPU

NPU

RAID

模块2：PCIe lane

PCIe是 多通道通信。

例如：

x1

x4

x8

x16

NVMe一般是：

PCIe x4

GPU一般是：

PCIe x16

模块3：PCIe Gen版本

不同版本速度不同：

| 版本 | 速度 |
| --- | --- |
| Gen3 | 8 GT/s |
| Gen4 | 16 GT/s |
| Gen5 | 32 GT/s |

你服务器：

PCIe Gen4

模块4：PCIe带宽计算

例如：

Gen4 x4

带宽：

≈ 8GB/s

公式（简单理解）：

lane × 单lane带宽

例如：

Gen4 x16 ≈ 32GB/s

模块5：识别PCIe设备

运行：

lspci

你服务器会看到：

NVMe controller

Ethernet controller

Processing accelerators

RAID controller

这些全部是 PCIe设备。

模块6：查看PCIe链路

运行：

lspci -vv

重点看：

LnkCap

LnkSta

例如：

Speed 16GT/s

Width x4

说明：

PCIe Gen4 x4

模块7：判断PCIe降速

如果：

LnkCap: 16GT/s

LnkSta: 8GT/s

说明：

PCIe降速

常见原因：

BIOS限制

设备兼容问题

信号问题

模块8：查看PCIe拓扑

运行：

lspci -tv

可以看到树结构：

CPU

└ Root Port

└ Switch

├ NVMe

├ NIC

└ NPU

模块9：PCIe Root Port

Root Port在：

CPU内部

作用：

连接CPU和PCIe设备

你服务器有 很多Root Port。

原因：

多PCIe设备

模块10：PCIe Switch

Switch作用：

扩展PCIe设备

例如：

1个x16

分成

4个x4

很多AI服务器都会用。

模块11：设备NUMA关系

运行：

cat /sys/bus/pci/devices/0000:81:00.0/numa_node

输出：

3

说明设备属于：

NUMA3

这对性能很重要。

模块12：Python自动检测PCIe

创建脚本：

import subprocess

result = subprocess.run(["lspci"],capture_output=True,text=True)

print("NVMe:", result.stdout.count("Non-Volatile"))

print("NIC:", result.stdout.count("Ethernet"))

print("NPU:", result.stdout.count("Processing"))

运行：

python3 check_pcie.py

Day4完成后能力

你会掌握：

PCIe架构

PCIe带宽

PCIe设备识别

PCIe降速判断

PCIe拓扑

NUMA关系

自动化检测

这其实就是 服务器硬件工程师核心能力之一。

但我其实想在 Day4 加一个 真正工程级训练

就是：

完整解析你服务器PCIe拓扑

我们会用：

lspci

lspci -tv

lspci -vv

把你服务器：

CPU

NUMA

Root Port

Switch

NVMe

RAID

NIC

NPU

全部画出来。

这一步很多 服务器工程师工作几年都不会做。

如果你愿意，下一步我可以直接带你做 Day4最关键的训练：

把你这台服务器完整 PCIe 拓扑图画出来。