服务器存储系统：NVMe / SATA / SAS / RAID / IO路径

1. 分清 NVMe、SATA、SAS、RAID 各自是什么

2. 看懂你这台机器的真实存储结构

3. 说清“数据是怎么从盘走到 CPU/NPU 的”

4. 建立第一版存储排查链

5. 写一个最基础的存储检测脚本

模块1：服务器存储体系总览

1、NVME:是nvme协议、PCIE总线、特点是低延迟、高带宽、多队列

2、SATA：AHCI\SATA协议、结构简单、速度比nvme慢、

如果系统lsblk看到sdx、可能是sata也可能是raid组成的逻辑盘

- SAS：企业级磁盘接口、常用于服务器、通常配合HBA和Raid控制器

- Raid：是磁盘控制层、把多块物理盘组合成逻辑盘、提供冗余、重建、缓存等能力、radi0、1、5、50、10

- 模块2：实机存储设备识别

- 本机两块U.2的NVME、一张raid卡、（一张bordcom/lsi卡）

- 应该是U.2形态的NVME  SSD直连pcie的盘

- L sblk-d 查看块设备查看NAME：sda、sdb、SIZE字段、TYPE

- [root@localhost ~]# lsblk -d

- NAME    MAJ:MIN RM   SIZE RO TYPE MOUNTPOINTS

- sda       8:0    0 447.1G  0 disk

- sdb       8:16   0 447.1G  0 disk

- sdc       8:32   0 446.6G  0 disk

- sdd       8:48   0 446.6G  0 disk

- sde       8:64   0 446.6G  0 disk

- sdf       8:80   0 446.6G  0 disk

- sdg       8:96   0 446.6G  0 disk

- sdh       8:112  0 446.6G  0 disk

- sdi       8:128  0 446.6G  0 disk

- sdj       8:144  0 446.6G  0 disk

- nvme1n1 259:0    0   2.9T  0 disk

- nvme0n1 259:1    0   2.9T  0 disk

- [root@localhost ~]# lsblk

- NAME               MAJ:MIN RM   SIZE RO TYPE MOUNTPOINTS

- sda                  8:0    0 447.1G  0 disk

- sdb                  8:16   0 447.1G  0 disk

- sdc                  8:32   0 446.6G  0 disk

- sdd                  8:48   0 446.6G  0 disk

- sde                  8:64   0 446.6G  0 disk

- sdf                  8:80   0 446.6G  0 disk

- sdg                  8:96   0 446.6G  0 disk

- sdh                  8:112  0 446.6G  0 disk

- sdi                  8:128  0 446.6G  0 disk

- sdj                  8:144  0 446.6G  0 disk

- nvme1n1            259:0    0   2.9T  0 disk

- ├─nvme1n1p1        259:5    0   600M  0 part /boot/efi

- ├─nvme1n1p2        259:6    0     1G  0 part /boot

- └─nvme1n1p3        259:7    0   2.9T  0 part

- ├─openeuler-root 253:0    0    70G  0 lvm  /

- ├─openeuler-swap 253:1    0     4G  0 lvm  [SWAP]

- └─openeuler-home 253:2    0   2.8T  0 lvm  /home

- nvme0n1            259:1    0   2.9T  0 disk

- ├─nvme0n1p1        259:2    0   600M  0 part

- ├─nvme0n1p2        259:3    0     1G  0 part

- └─nvme0n1p3        259:4    0   2.9T  0 part

- ├─klas-swap      253:3    0     4G  0 lvm

- ├─klas-backup    253:4    0    50G  0 lvm

- └─klas-root      253:5    0   2.9T  0 lvm

- Nvme list：设备名、容量、型号、固件版本

- [root@localhost ~]# nvme list

- Node                  SN                   Model                                    Namespace Usage                      Format           FW Rev

- --------------------- -------------------- ---------------------------------------- --------- -------------------------- ---------------- --------

- /dev/nvme0n1          D77446D401J852       DERAP44YGM03T2US                         1           3.20  TB /   3.20  TB    512   B +  0 B   D7Y05M1F

- /dev/nvme1n1          D77446D4017E52       DERAP44YGM03T2US                         1           3.20  TB /   3.20  TB    512   B +  0 B   D7Y05M1F

- 查看scsi/raid后面的盘

- [root@localhost ~]# lsblk -d

- NAME    MAJ:MIN RM   SIZE RO TYPE MOUNTPOINTS

- sda       8:0    0 447.1G  0 disk

- sdb       8:16   0 447.1G  0 disk

- sdc       8:32   0 446.6G  0 disk

- sdd       8:48   0 446.6G  0 disk

- sde       8:64   0 446.6G  0 disk

- sdf       8:80   0 446.6G  0 disk

- sdg       8:96   0 446.6G  0 disk

- sdh       8:112  0 446.6G  0 disk

- sdi       8:128  0 446.6G  0 disk

- sdj       8:144  0 446.6G  0 disk

- nvme1n1 259:0    0   2.9T  0 disk

- nvme0n1 259:1    0   2.9T  0 disk

- [root@localhost ~]# lsscsi

- [0:1:124:0]  enclosu BROADCOM VirtualSES       03    -

- [0:3:104:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdc

- [0:3:105:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdd

- [0:3:106:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sde

- [0:3:107:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdf

- [0:3:108:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdi

- [0:3:109:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdj

- [0:3:110:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdh

- [0:3:111:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdg

- [1:0:0:0]    disk    ATA      INTEL SSDSCKKB48 0100  /dev/sda

- [2:0:0:0]    disk    ATA      INTEL SSDSCKKB48 0100  /dev/sdb

- [N:0:0:1]    disk    DERAP44YGM03T2US__1                        /dev/nvme0n1

- [N:1:0:1]    disk    DERAP44YGM03T2US__1                        /dev/nvme1n1

- [root@localhost ~]#

- 哪些盘来自 RAID 控制器

- 哪些是单独直通的设备

- 看 PCIe 层的存储控制器

- [root@localhost ~]# lspci | egrep -i "SATA|SAS|NVME|SCSI|raid"

- 30:04.0 Serial Attached SCSI controller: Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA (rev 30)

- 32:04.0 Serial Attached SCSI controller: Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA (rev 30)

- 38:05.0 SATA controller: Huawei Technologies Co., Ltd. HiSilicon AHCI HBA (rev 30)

- 70:04.0 Serial Attached SCSI controller: Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA (rev 30)

- 72:04.0 Serial Attached SCSI controller: Huawei Technologies Co., Ltd. HiSilicon SAS 3.0 HBA (rev 30)

- 83:00.0 RAID bus controller: Broadcom / LSI MegaRAID 12GSAS/PCIe Secure SAS39xx

- [root@localhost ~]#

- 模块3：NVMe / SATA / SAS / RAID 区别

- 看到 nvme0n1 / nvme1n1这是NVMe 块设备本质是pcie设备

- Nvme list、lspci | grep -i nvme（ Non-Volatile memory ）

- [root@localhost ~]# lspci | egrep -i "Non"

- 81:00.0 Non-Volatile memory controller: DERA Storage Device 1515

- 82:00.0 Non-Volatile memory controller: DERA Storage Device 1515

- [root@localhost ~]#

- 看到sdx、不一定是sata盘、继续看lsscsi、看他属于哪个设备

- 是raid的逻辑盘还是直通设备

如果它显示：

BROADCOM MR9560-16i

这种 RAID 控制器信息，就说明：

这是 RAID 逻辑盘 / RAID 控制器后面的盘

- 设备一：

- root@thtf-pc:~# lsscsi

- [N:0:0:1]    disk    BIWIN AP443 1TB SSD__1                     /dev/nvme0n1

- root@thtf-pc:~#

- 设备二：

- [root@localhost ~]# lsscsi

- [0:1:124:0]  enclosu BROADCOM VirtualSES       03    -

- [0:3:104:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdc

- [0:3:105:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdd

- [0:3:106:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sde

- [0:3:107:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdf

- [0:3:108:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdi

- [0:3:109:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdj

- [0:3:110:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdh

- [0:3:111:0]  disk    BROADCOM MR9560-16i       5.29  /dev/sdg

- [1:0:0:0]    disk    ATA      INTEL SSDSCKKB48 0100  /dev/sda

- [2:0:0:0]    disk    ATA      INTEL SSDSCKKB48 0100  /dev/sdb

- [N:0:0:1]    disk    DERAP44YGM03T2US__1                        /dev/nvme0n1

- [N:1:0:1]    disk    DERAP44YGM03T2US__1                        /dev/nvme1n1

- [root@localhost ~]#

- 看到ATA 是sata设备、

- 结合机器BOM验证sda、sdb是M.2形态的SATA协议的设备

- NVMe 是盘类型 + 协议
RAID 是控制层
SATA / SAS 是磁盘接口体系

- 看到 RAID bus controller

- 说明这是：RAID 控制器本身

- 设备三：[root@bogon ~]# lsscsi

- [0:0:7:0]    disk    ATA      WUS721208BLE604  WNFB  /dev/sda

- [0:2:0:0]    disk    HUAWEI   RAID0            C00   /dev/sdb

- [0:2:1:0]    disk    HUAWEI   RAID0            C00   /dev/sdc

- [0:2:2:0]    disk    HUAWEI   RAID1            C00   /dev/sdd

- [0:2:3:0]    disk    HUAWEI   RAID5            C00   /dev/sde

- [0:2:4:0]    disk    HUAWEI   RAID0            C00   /dev/sdf

- [0:2:5:0]    disk    HUAWEI   RAID1            C00   /dev/sdg

- [root@bogon ~]#

- 模块4：你这台机器的存储路径图

- 6U这台机器的存储结构可以先画成这样

- CPU
└─ PCIe Root Port
	├─ NVMe (U.2) ×2
	└─ RAID Controller
		└─ 多块 SSD（SAS / SATA 体系）

这张图非常重要，因为它直接决定：

带宽
延迟
故障位置
排查顺序

以后你看到“存储慢”，不能只说：

磁盘慢

你要能继续往下追：

是 NVMe 本身慢？
是 RAID 层慢？
是 PCIe 上游链路有竞争？
是跨 NUMA？

这就是系统工程思维。

- 模块5：存储异常排查链

1. 如果“盘不见了”

NVMe 不见了

lsblk
→ nvme list
→ lspci | grep -i nvme
→ dmesg | grep -i nvme
→ lsmod | grep nvme

RAID / SAS 盘异常

lsblk
→ lsscsi
→ storcli
→ dmesg
→ lspci | grep -i raid

NVMe：块设备 → 控制器 → PCIe → 日志 → 驱动
RAID：块设备 → SCSI/RAID → 管理工具 → 日志 → PCIe

2. 如果“盘还在，但是慢”

先看：

iostat -x 1

然后区分：

%util 高不高

await 高不高

是 NVMe 慢还是 RAID 慢

有没有上游链路共享问题

- 模块6：Python存储检测脚本

让脚本帮你把存储设备列出来

会用 subprocess 拉系统信息

Vi storage_check.py

import subprocess

print("==== lsblk ====")

print(subprocess.run(["lsblk", "-d"], capture_output=True, text=True).stdout)

print("==== nvme list ====")

print(subprocess.run(["nvme", "list"], capture_output=True, text=True).stdout)

print("==== lsscsi ====")

print(subprocess.run(["lsscsi"], capture_output=True, text=True).stdout)

python3 storage_check.py

M.2 ，U.2是形态

SATA / NVMe 是协议

PCIe 是总线

Storcli64使用方法（博通）

storcli 是“先看控制器 /c0，再看逻辑盘 /vall，再看物理盘 /eall/sall”

# 1. 先看控制器在不在

storcli show

[root@localhost ~]# storcli64 show

CLI Version = 007.2707.0000.0000 Dec 18, 2023

Operating system = Linux 5.10.0-216.0.0.115.oe2203sp4.aarch64

Status Code = 0

Status = Success

Description = None

Number of Controllers = 1

Host Name = localhost.localdomain

Operating System  = Linux 5.10.0-216.0.0.115.oe2203sp4.aarch64

System Overview :

===============

--------------------------------------------------------------------------------

Ctl Model               Ports PDs DGs DNOpt VDs VNOpt BBU sPR DS  EHS ASOs Hlth

--------------------------------------------------------------------------------

0 MegaRAID9560-16i8GB    16   8   8     0   8     0 N/A On  1&2 Y      4 Opt

--------------------------------------------------------------------------------

Ctl=Controller Index|DGs=Drive groups|VDs=Virtual drives|Fld=Failed

PDs=Physical drives|DNOpt=Array NotOptimal|VNOpt=VD NotOptimal|Opt=Optimal

Msng=Missing|Dgd=Degraded|NdAtn=Need Attention|Unkwn=Unknown

sPR=Scheduled Patrol Read|DS=DimmerSwitch|EHS=Emergency Spare Drive

Y=Yes|N=No|ASOs=Advanced Software Options|BBU=Battery backup unit/CV

Hlth=Health|Safe=Safe-mode boot|CertProv-Certificate Provision mode

Chrg=Charging | MsngCbl=Cable Failure

[root@localhost ~]#

2. 看 c0 总览

storcli /c0 show

[root@localhost ~]# storcli64 /c0 show

Generating detailed summary of the adapter, it may take a while to complete.

CLI Version = 007.2707.0000.0000 Dec 18, 2023

Operating system = Linux 5.10.0-216.0.0.115.oe2203sp4.aarch64

Controller = 0

Status = Success

Description = None

Product Name = MegaRAID 9560-16i 8GB

Serial Number = SPE5201509

SAS Address =  500062b220902e00

PCI Address = 00:83:00:00

System Time = 03/16/2026 15:31:06

Mfg. Date = 01/10/25

Controller Time = 03/16/2026 07:31:05

FW Package Build = 52.29.0-5442

BIOS Version = 7.29.00.0_0x071D0000

FW Version = 5.290.02-3997

Driver Name = megaraid_sas

Driver Version = 07.714.04.00-rc1

Current Personality = RAID-Mode

Vendor Id = 0x1000

Device Id = 0x10E2

SubVendor Id = 0x1000

SubDevice Id = 0x4000

Host Interface = PCI-E

Device Interface = SAS-12G

Bus Number = 131

Device Number = 0

Function Number = 0

Domain ID = 0

Security Protocol = None

Drive Groups = 8

TOPOLOGY :

========

-----------------------------------------------------------------------------

DG Arr Row EID:Slot DID Type  State BT       Size PDC  PI SED DS3  FSpace TR

-----------------------------------------------------------------------------

0 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

0 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

0 0   0   252:0    2   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

1 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

1 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

1 0   0   252:1    9   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

2 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

2 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

2 0   0   252:3    5   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

3 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

3 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

3 0   0   252:4    4   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

4 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

4 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

4 0   0   252:5    6   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

5 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

5 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

5 0   0   252:6    7   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

6 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

6 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

6 0   0   252:7    8   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

7 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

7 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

7 0   0   252:2    3   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

-----------------------------------------------------------------------------

DG=Disk Group Index|Arr=Array Index|Row=Row Index|EID=Enclosure Device ID

DID=Device ID|Type=Drive or RAID Type|Onln=Online|Rbld=Rebuild|Optl=Optimal

Dgrd=Degraded|Pdgd=Partially degraded|Offln=Offline|BT=Background Task Active

PDC=PD Cache|PI=Protection Info|SED=Self Encrypting Drive|Frgn=Foreign

DS3=Dimmer Switch 3|dflt=Default|Msng=Missing|FSpace=Free Space Present

TR=Transport Ready

Virtual Drives = 8

VD LIST :

=======

---------------------------------------------------------------

DG/VD TYPE  State Access Consist Cache Cac sCC       Size Name

---------------------------------------------------------------

6/232 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

5/233 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

4/234 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

3/235 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

2/236 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

7/237 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

1/238 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

0/239 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

---------------------------------------------------------------

VD=Virtual Drive| DG=Drive Group|Rec=Recovery

Cac=CacheCade|OfLn=OffLine|Pdgd=Partially Degraded|Dgrd=Degraded

Optl=Optimal|dflt=Default|RO=Read Only|RW=Read Write|HD=Hidden|TRANS=TransportReady

B=Blocked|Consist=Consistent|R=Read Ahead Always|NR=No Read Ahead|WB=WriteBack

AWB=Always WriteBack|WT=WriteThrough|C=Cached IO|D=Direct IO|sCC=Scheduled

Check Consistency

Physical Drives = 8

PD LIST :

=======

------------------------------------------------------------------------------

EID:Slt DID State DG       Size Intf Med SED PI SeSz Model            Sp Type

------------------------------------------------------------------------------

252:0     2 Onln   0 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:1     9 Onln   1 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:2     3 Onln   7 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:3     5 Onln   2 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:4     4 Onln   3 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:5     6 Onln   4 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:6     7 Onln   5 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:7     8 Onln   6 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

------------------------------------------------------------------------------

EID=Enclosure Device ID|Slt=Slot No|DID=Device ID|DG=DriveGroup

DHS=Dedicated Hot Spare|UGood=Unconfigured Good|GHS=Global Hotspare

UBad=Unconfigured Bad|Sntze=Sanitize|Onln=Online|Offln=Offline|Intf=Interface

Med=Media Type|SED=Self Encryptive Drive|PI=PI Eligible

SeSz=Sector Size|Sp=Spun|U=Up|D=Down|T=Transition|F=Foreign

UGUnsp=UGood Unsupported|UGShld=UGood shielded|HSPShld=Hotspare shielded

CFShld=Configured shielded|Cpybck=CopyBack|CBShld=Copyback Shielded

UBUnsp=UBad Unsupported|Rbld=Rebuild

Enclosures = 1

Enclosure LIST :

==============

------------------------------------------------------------------------

EID State Slots PD PS Fans TSs Alms SIM Port# ProdID     VendorSpecific

------------------------------------------------------------------------

252 OK       16  8  0    0   0    0   0 -     VirtualSES

------------------------------------------------------------------------

EID=Enclosure Device ID | PD=Physical drive count | PS=Power Supply count

TSs=Temperature sensor count | Alms=Alarm count | SIM=SIM Count | ProdID=Product ID

[root@localhost ~]#

# 3. 看所有逻辑盘

Storcli64 /c0/vall show

[root@localhost ~]# storcli64 /c0/vall show

CLI Version = 007.2707.0000.0000 Dec 18, 2023

Operating system = Linux 5.10.0-216.0.0.115.oe2203sp4.aarch64

Controller = 0

Status = Success

Description = None

Virtual Drives :

==============

---------------------------------------------------------------

DG/VD TYPE  State Access Consist Cache Cac sCC       Size Name

---------------------------------------------------------------

6/232 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

5/233 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

4/234 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

3/235 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

2/236 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

7/237 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

1/238 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

0/239 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

---------------------------------------------------------------

VD=Virtual Drive| DG=Drive Group|Rec=Recovery

Cac=CacheCade|OfLn=OffLine|Pdgd=Partially Degraded|Dgrd=Degraded

Optl=Optimal|dflt=Default|RO=Read Only|RW=Read Write|HD=Hidden|TRANS=TransportReady

B=Blocked|Consist=Consistent|R=Read Ahead Always|NR=No Read Ahead|WB=WriteBack

AWB=Always WriteBack|WT=WriteThrough|C=Cached IO|D=Direct IO|sCC=Scheduled

Check Consistency

[root@localhost ~]#

# 4. 看所有物理盘

storcli /c0/eall/sall show

[root@localhost ~]# storcli64 /c0/eall/sall show

CLI Version = 007.2707.0000.0000 Dec 18, 2023

Operating system = Linux 5.10.0-216.0.0.115.oe2203sp4.aarch64

Controller = 0

Status = Success

Description = Show Drive Information Succeeded.

Drive Information :

=================

------------------------------------------------------------------------------

EID:Slt DID State DG       Size Intf Med SED PI SeSz Model            Sp Type

------------------------------------------------------------------------------

252:0     2 Onln   0 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:1     9 Onln   1 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:2     3 Onln   7 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:3     5 Onln   2 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:4     4 Onln   3 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:5     6 Onln   4 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:6     7 Onln   5 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:7     8 Onln   6 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

------------------------------------------------------------------------------

EID=Enclosure Device ID|Slt=Slot No|DID=Device ID|DG=DriveGroup

DHS=Dedicated Hot Spare|UGood=Unconfigured Good|GHS=Global Hotspare

UBad=Unconfigured Bad|Sntze=Sanitize|Onln=Online|Offln=Offline|Intf=Interface

Med=Media Type|SED=Self Encryptive Drive|PI=PI Eligible

SeSz=Sector Size|Sp=Spun|U=Up|D=Down|T=Transition|F=Foreign

UGUnsp=UGood Unsupported|UGShld=UGood shielded|HSPShld=Hotspare shielded

CFShld=Configured shielded|Cpybck=CopyBack|CBShld=Copyback Shielded

UBUnsp=UBad Unsupported|Rbld=Rebuild

[root@localhost ~]#

# 5. 想看更细再展开

storcli /c0 show all

华为有自己的工具

Show：这会列出控制器列表；StorCLI 的对象是按控制器编号来看的，后面常见就是 /c0。StorCLI 的通用语法也是按 /cX 这种控制器索引来组织的。

storcli /c0 show all

查看更详细的信息

[root@localhost ~]# storcli64 /c0 show all

Generating detailed summary of the adapter, it may take a while to complete.

CLI Version = 007.2707.0000.0000 Dec 18, 2023

Operating system = Linux 5.10.0-216.0.0.115.oe2203sp4.aarch64

Controller = 0

Status = Success

Description = None

Basics :

======

Controller = 0

Model = MegaRAID 9560-16i 8GB

Serial Number = SPE5201509

Current Controller Date/Time = 03/16/2026, 07:35:14

Current System Date/time = 03/16/2026, 15:35:14

SAS Address = 500062b220902e00

PCI Address = 00:83:00:00

Mfg Date = 01/10/25

Rework Date = 00/00/00

Revision No = 00011

Version :

=======

Firmware Package Build = 52.29.0-5442

Firmware Version = 5.290.02-3997

PSOC FW Version = 0x001B

PSOC Hardware Version = 0x000A

PSOC Part Number = 29211-270-8GB

NVDATA Version = 5.2900.00-0761

CBB Version = 29.250.01.00

Bios Version = 7.29.00.0_0x071D0000

HII Version = 07.29.00.00

HIIA Version = 07.29.00.00

Driver Name = megaraid_sas

Driver Version = 07.714.04.00-rc1

Bus :

===

Vendor Id = 0x1000

Device Id = 0x10E2

SubVendor Id = 0x1000

SubDevice Id = 0x4000

Host Interface = PCI-E

Device Interface = SAS-12G

Bus Number = 131

Device Number = 0

Function Number = 0

Domain ID = 0

Pending Images in Flash :

=======================

Image name = No pending images

Status :

======

Controller Status = Optimal

Memory Correctable Errors = 0

Memory Uncorrectable Errors = 0

ECC Bucket Count = 0

Any Offline VD Cache Preserved = No

BBU Status = NA

PD Firmware Download in progress = No

Support PD Firmware Download = Yes

Lock Key Assigned = No

Failed to get lock key on bootup = No

Lock key has not been backed up = No

Bios was not detected during boot = No

Controller must be rebooted to complete security operation = No

A rollback operation is in progress = No

At least one PFK exists in NVRAM = No

SSC Policy is WB = No

Controller has booted into safe mode = No

Controller shutdown required = No

Controller has booted into certificate provision mode = No

Current Personality = RAID-Mode

Supported Adapter Operations :

============================

Rebuild Rate = Yes

CC Rate = Yes

BGI Rate  = Yes

Reconstruct Rate = Yes

Patrol Read Rate = Yes

Alarm Control = No

Cluster Support = No

BBU = NA

Spanning = Yes

Dedicated Hot Spare = Yes

Revertible Hot Spares = Yes

Foreign Config Import = Yes

Self Diagnostic = Yes

Allow Mixed Redundancy on Array = No

Global Hot Spares = Yes

Deny SCSI Passthrough = No

Deny SMP Passthrough = No

Deny STP Passthrough = No

Support more than 8 Phys = Yes

FW and Event Time in GMT = No

Support Enhanced Foreign Import = Yes

Support Enclosure Enumeration = Yes

Support Allowed Operations = Yes

Abort CC on Error = Yes

Support Multipath = Yes

Support Odd & Even Drive count in RAID1E = No

Support Security = Yes

Support Config Page Model = Yes

Support the OCE without adding drives = Yes

Support EKM = Yes

Snapshot Enabled = No

Support PFK = Yes

Support PI = No

Support Ld BBM Info = No

Support Shield State = Yes

Block SSD Write Disk Cache Change = No

Support Suspend Resume BG ops = Yes

Support Emergency Spares = Yes

Support Set Link Speed = Yes

Support Boot Time PFK Change = No

Support JBOD = No

Disable Online PFK Change = No

Support Perf Tuning = Yes

Support SSD PatrolRead = Yes

Real Time Scheduler = Yes

Support Reset Now = Yes

Support Emulated Drives = Yes

Headless Mode = Yes

Dedicated HotSpares Limited = No

Point In Time Progress = Yes

Extended LD = Yes

Support Uneven span  = No

Support Config Auto Balance = No

Support Maintenance Mode = No

Support Diagnostic results = Yes

Support Ext Enclosure = Yes

Support Sesmonitoring = Yes

Support SecurityonJBOD = Yes

Support ForceFlash = Yes

Support DisableImmediateIO = Yes

Support LargeIOSupport = No

Support DrvActivityLEDSetting = Yes

Support FlushWriteVerify = Yes

Support CPLDUpdate = No

Support ForceTo512e = Yes

Support discardCacheDuringLDDelete = No

Support JBOD Write cache = No

Support Large QD Support = No

Support Ctrl Info Extended = Yes

Support IButton less = Yes

Support AES Encryption Algorithm = No

Support Encrypted MFC = Yes

Support Snapdump = Yes

Support Force Personality Change = Yes

Support Dual Fw Image = No

Support PSOC Update = Yes

Support Secure Boot = Yes

Support Debug Queue = No

Support Least Latency Mode = No

Support OnDemand Snapdump = Yes

Support Clear Snapdump = Yes

Support FW Triggered Snapdump = Yes

Support PHY current speed = Yes

Support Lane current speed = Yes

Support NVMe Width = Yes

Support Lane DeviceType = Yes

Support Extended Drive performance Monitoring = Yes

Support NVMe Repair = Yes

Support Platform Security = No

Support None Mode Params = Yes

Support Extended Controller Property = Yes

Support Smart Poll Interval for DirectAttached = Yes

Support Write Journal Pinning = Yes

Support SMP Passthru with Port Number = Yes

Support SnapDump Preboot Trace Buffer Toggle = Yes

Support Parity Read Cache Bypass = Yes

Support NVMe Init Error Device ConnectorIndex = No

Support VolatileKey = Yes

Support PSOC Part Information = Yes

Support Slow array threshold calculation = Yes

Support PCIe Reference Clock override = Yes

Support Target ID Reuse = Yes

Support PCIe PERST override = Yes

Support Drive FW Download Mask = Yes

Support Drive FW Download Mode-5 = Yes

Support Drive FW Download Mode-7 = Yes

Support Drive FW Download Mode-E = Yes

Supported Drive FW Download Chunk Size = 32 KB

Enterprise Key management :

=========================

Capability = Supported

Boot Agent = Not Available

Configured = No

Supported PD Operations :

=======================

Force Online = Yes

Force Offline = Yes

Force Rebuild = Yes

Deny Force Failed = No

Deny Force Good/Bad = No

Deny Missing Replace = No

Deny Clear = No

Deny Locate = No

Support Power State = Yes

Set Power State For Cfg = No

Support T10 Power State = No

Support Temperature = Yes

NCQ = Yes

Support Max Rate SATA = No

Support Degraded Media = No

Support Parallel FW Update = Yes

Support Drive Crypto Erase = Yes

Support SSD Wear Gauge = No

Support Sanitize = No

Support Extended Sanitize = Yes

Supported VD Operations :

=======================

Read Policy = Yes

Write Policy = Yes

IO Policy = No

Access Policy = Yes

Disk Cache Policy = Yes

Reconstruction = Yes

Deny Locate = No

Deny CC = No

Allow Ctrl Encryption = No

Enable LDBBM = Yes

Support FastPath = Yes

Performance Metrics = Yes

Power Savings = No

Support Powersave Max With Cache = No

Support Breakmirror = No

Support SSC WriteBack = No

Support SSC Association = No

Support VD Hide = Yes

Support VD Cachebypass = Yes

Support VD discardCacheDuringLDDelete = No

Support VD Scsi Unmap = Yes

Advanced Software Option :

========================

-----------------------------------------

Adv S/W Opt         Time Remaining  Mode

-----------------------------------------

MegaRAID FastPath   Unlimited       -

MegaRAID SafeStore  Unlimited       -

MegaRAID RAID6      Unlimited       -

MegaRAID RAID5      Unlimited       -

-----------------------------------------

Safe ID =  B94DI8GWNMJ8R5MPE8NWERM14D1LDVCN83613RAZ

HwCfg :

=====

ChipRevision =  A0

BatteryFRU = N/A

Front End Port Count = 0

Backend Port Count = 16

BBU = Absent

Alarm = Absent

Serial Debugger = Present

NVRAM Size = 128KB

Flash Size = 16MB

On Board Memory Size = 8192MB

CacheVault Flash Size = NA

TPM = Absent

Upgrade Key = Absent

On Board Expander = Absent

Temperature Sensor for ROC = Present

Temperature Sensor for Controller = Absent

Upgradable CPLD = Absent

Upgradable PSOC = Present

Current Size of CacheCade (GB) = 0

Current Size of FW Cache (MB) = 6678

ROC temperature(Degree Celsius) = 42

Policies :

========

Policies Table :

==============

------------------------------------------------

Policy                          Current Default

------------------------------------------------

Predictive Fail Poll Interval   300 sec

Interrupt Throttle Active Count 16

Interrupt Throttle Completion   50 us

Rebuild Rate                    30 %    30%

PR Rate                         30 %    30%

BGI Rate                        30 %    30%

Check Consistency Rate          30 %    30%

Reconstruction Rate             30 %    30%

Cache Flush Interval            4s

------------------------------------------------

Flush Time(Default) = 4s

Drive Coercion Mode = none

Auto Rebuild = On

Battery Warning = On

ECC Bucket Size = 15

ECC Bucket Leak Rate (hrs) = 24

Restore Hot Spare on Insertion = Off

Expose Enclosure Devices = On

Maintain PD Fail History = On

Reorder Host Requests = On

Auto detect BackPlane = SGPIO/i2c SEP

Load Balance Mode = Auto

Security Key Assigned = Off

Disable Online Controller Reset = Off

Use drive activity for locate = Off

Boot :

====

BIOS Enumerate VDs = 1

Stop BIOS on Error = Off

Delay during POST = 0

Spin Down Mode = None

Enable Ctrl-R = No

Enable Web BIOS = No

Enable PreBoot CLI = No

Enable BIOS = Yes

Max Drives to Spinup at One Time = 2

Maximum number of direct attached drives to spin up in 1 min = 60

Delay Among Spinup Groups (sec) = 2

Allow Boot with Preserved Cache = Off

High Availability :

=================

Topology Type = None

Cluster Permitted = No

Cluster Active = No

Defaults :

========

Phy Polarity = 0

Phy PolaritySplit = 0

Strip Size = 256 KB

Write Policy = WB

Read Policy = RA

Cache When BBU Bad = Off

Cached IO = Off

VD PowerSave Policy = Controller Defined

Default spin down time (mins) = 30

Coercion Mode = None

ZCR Config = Unknown

Max Chained Enclosures = 16

Direct PD Mapping = No

Restore Hot Spare on Insertion = No

Expose Enclosure Devices = Yes

Maintain PD Fail History = Yes

Zero Based Enclosure Enumeration = No

Disable Puncturing = No

EnableLDBBM = Yes

DisableHII = No

Un-Certified Hard Disk Drives = Allow

SMART Mode = Mode 6

Enable LED Header = Yes

LED Show Drive Activity = Yes

Dirty LED Shows Drive Activity = No

EnableCrashDump = No

Disable Online Controller Reset = No

Treat Single span R1E as R10 = No

Power Saving option = Enabled

TTY Log In Flash = No

Auto Enhanced Import = Yes

BreakMirror RAID Support = No

Disable Join Mirror = Yes

Enable Shield State = Yes

Time taken to detect CME = 60 sec

Capabilities :

============

Supported Drives = SAS, SATA, NVMe

RAID Level Supported = RAID0, RAID1(2 or more drives), RAID5, RAID6, RAID00, RAID10(2 or more drives per span), RAID50, RAID60

Enable JBOD = No

Mix in Enclosure = Allowed

Mix of SAS/SATA of HDD type in VD = Allowed

Mix of SAS/SATA of SSD type in VD = Not Allowed

Mix of SSD/HDD in VD = Not Allowed

SAS Disable = No

Max Arms Per VD = 32

Max Spans Per VD = 8

Max Arrays = 240

Max VD per array = 16

Max Number of VDs = 240

Max Parallel Commands = 5101

Max SGE Count = 60

Max Data Transfer Size = 2048 sectors

Max Strips PerIO = 42

Max Configurable CacheCade Size(GB) = 0

Max Transportable DGs = 0

Enable Snapdump = Yes

Enable SCSI Unmap = Yes

Read cache bypass enabled for Parity RAID LDs = Yes

FDE Drive Mix Support = No

Min Strip Size = 64 KB

Max Strip Size = 1.000 MB

Scheduled Tasks :

===============

Consistency Check Reoccurrence = 168 hrs

Next Consistency check launch = 03/21/2026, 03:00:00

Patrol Read Reoccurrence = 168 hrs

Next Patrol Read launch = 03/21/2026, 03:00:00

Battery learn Reoccurrence = NA

Next Battery Learn = NA

OEMID = Broadcom

Secure Boot :

===========

Secure Boot Enabled = Yes

Controller in Soft Secure Mode = No

Controller in Hard Secure Mode = Yes

Key Update Pending = No

Remaining Secure Boot Key Slots = 7

Security Protocol properties :

============================

Security Protocol = None

Drive Groups = 8

TOPOLOGY :

========

-----------------------------------------------------------------------------

DG Arr Row EID:Slot DID Type  State BT       Size PDC  PI SED DS3  FSpace TR

-----------------------------------------------------------------------------

0 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

0 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

0 0   0   252:0    2   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

1 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

1 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

1 0   0   252:1    9   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

2 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

2 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

2 0   0   252:3    5   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

3 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

3 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

3 0   0   252:4    4   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

4 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

4 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

4 0   0   252:5    6   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

5 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

5 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

5 0   0   252:6    7   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

6 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

6 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

6 0   0   252:7    8   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

7 -   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

7 0   -   -        -   RAID0 Optl  N  446.625 GB dflt N  N   none N      N

7 0   0   252:2    3   DRIVE Onln  N  446.625 GB dflt N  N   none -      N

-----------------------------------------------------------------------------

DG=Disk Group Index|Arr=Array Index|Row=Row Index|EID=Enclosure Device ID

DID=Device ID|Type=Drive or RAID Type|Onln=Online|Rbld=Rebuild|Optl=Optimal

Dgrd=Degraded|Pdgd=Partially degraded|Offln=Offline|BT=Background Task Active

PDC=PD Cache|PI=Protection Info|SED=Self Encrypting Drive|Frgn=Foreign

DS3=Dimmer Switch 3|dflt=Default|Msng=Missing|FSpace=Free Space Present

TR=Transport Ready

Virtual Drives = 8

VD LIST :

=======

---------------------------------------------------------------

DG/VD TYPE  State Access Consist Cache Cac sCC       Size Name

---------------------------------------------------------------

6/232 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

5/233 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

4/234 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

3/235 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

2/236 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

7/237 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

1/238 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

0/239 RAID0 Optl  RW     Yes     RWTD  -   ON  446.625 GB

---------------------------------------------------------------

VD=Virtual Drive| DG=Drive Group|Rec=Recovery

Cac=CacheCade|OfLn=OffLine|Pdgd=Partially Degraded|Dgrd=Degraded

Optl=Optimal|dflt=Default|RO=Read Only|RW=Read Write|HD=Hidden|TRANS=TransportReady

B=Blocked|Consist=Consistent|R=Read Ahead Always|NR=No Read Ahead|WB=WriteBack

AWB=Always WriteBack|WT=WriteThrough|C=Cached IO|D=Direct IO|sCC=Scheduled

Check Consistency

Physical Drives = 8

PD LIST :

=======

------------------------------------------------------------------------------

EID:Slt DID State DG       Size Intf Med SED PI SeSz Model            Sp Type

------------------------------------------------------------------------------

252:0     2 Onln   0 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:1     9 Onln   1 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:2     3 Onln   7 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:3     5 Onln   2 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:4     4 Onln   3 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:5     6 Onln   4 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:6     7 Onln   5 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

252:7     8 Onln   6 446.625 GB SATA SSD N   N  512B HWE62ST3480L003N U  -

------------------------------------------------------------------------------

EID=Enclosure Device ID|Slt=Slot No|DID=Device ID|DG=DriveGroup

DHS=Dedicated Hot Spare|UGood=Unconfigured Good|GHS=Global Hotspare

UBad=Unconfigured Bad|Sntze=Sanitize|Onln=Online|Offln=Offline|Intf=Interface

Med=Media Type|SED=Self Encryptive Drive|PI=PI Eligible

SeSz=Sector Size|Sp=Spun|U=Up|D=Down|T=Transition|F=Foreign

UGUnsp=UGood Unsupported|UGShld=UGood shielded|HSPShld=Hotspare shielded

CFShld=Configured shielded|Cpybck=CopyBack|CBShld=Copyback Shielded

UBUnsp=UBad Unsupported|Rbld=Rebuild

Enclosures = 1

Enclosure LIST :

==============

------------------------------------------------------------------------

EID State Slots PD PS Fans TSs Alms SIM Port# ProdID     VendorSpecific

------------------------------------------------------------------------

252 OK       16  8  0    0   0    0   0 -     VirtualSES

------------------------------------------------------------------------

EID=Enclosure Device ID | PD=Physical drive count | PS=Power Supply count

TSs=Temperature sensor count | Alms=Alarm count | SIM=SIM Count | ProdID=Product ID

[root@localhost ~]#

华为Hiraidadm用法

[root@bogon ~]# ./hiraidadm show allctrl

========================================== Hiraidadm   Information ===========================================

Name: hiraidadm

Version: 1.2.2.9

Build Time: Jul  4 2025 11:03:05

==============================================================================================================

==============================================================================================================

Status Code = 0

Status = Success

Description = None

==============================================================================================================

Controllers Number = 1

-------------------------------------------------------

Controller Id           | 0

Device Name             | /dev/bsg/hiraid0

Pcie Bus Domain         | 0000

Bus Number              | 18

Device Number           | 00

Function Number         | 00

PCI Address             | 0000:18:00.0

[root@bogon ~]# ./hiraidadm show ctrlcount

========================================== Hiraidadm   Information ===========================================

Name: hiraidadm

Version: 1.2.2.9

Build Time: Jul  4 2025 11:03:05

==============================================================================================================

==============================================================================================================

Status Code = 0

Status = Success

Description = None

==============================================================================================================

ctrlcount:1

==============================================================================================================

[root@bogon ~]# hiraidadm c0 show config

-bash: hiraidadm：未找到命令

[root@bogon ~]# ./hiraidadm c0 show config

========================================== Hiraidadm   Information ===========================================

Name: hiraidadm

Version: 1.2.2.9

Build Time: Jul  4 2025 11:03:05

==============================================================================================================

==============================================================================================================

Status Code = 0

Status = Success

Description = None

==============================================================================================================

--------------------------------------------------------------------------------------------------------------

Raid Global Config Information

--------------------------------------------------------------------------------------------------------------

Precopy Switch             | On

Emergency Hotspare Switch  | Off

CopyBack Switch            | On

Bginit Rate                | low

Rebuild Rate               | high

CopyBack Rate              | high

PreCopy Rate               | high

Sanitize Rate              | low

CCheck Switch              | On

CCheck Period              | 30 day

CCheck Rate                | low

CCheck Repair Switch       | On

Patrolread Switch          | On

Patrolread Period          | 30 day

Patrolread Rate            | low

Drive Parall Num           | 4

Auto Import Fncfg Switch   | Off

--------------------------------------------------------------------------------------------------------------

[root@bogon ~]# ./hiraidadm c0 show pdlist

========================================== Hiraidadm   Information ===========================================

Name: hiraidadm

Version: 1.2.2.9

Build Time: Jul  4 2025 11:03:05

==============================================================================================================

==============================================================================================================

Status Code = 0

Status = Success

Description = None

==============================================================================================================

Total pd number is 12

--------------------------------------------------------------------------------------------------------------

Index Enc  Slot DID  Intf  Media Type     Status         PreFail SlowDrive LinkSpd Capacity   SecType FUA SED Spin FW       Model

--------------------------------------------------------------------------------------------------------------

0     0    0    1    SATA  SSD   member   online         No      No        6.0Gb/s   447.132GB  512E   Sup No  UP   6060     HWE74ST3480L007N

1     0    1    0    SATA  SSD   member   online         No      No        6.0Gb/s   447.132GB  512E   Sup No  UP   6060     HWE74ST3480L007N

2     0    2    4    SATA  HDD   member   online         No      No        6.0Gb/s     7.277TB  512E   Sup No  UP   V1GAWNFB WUS721208BLE604

3     0    3    5    SATA  HDD   member   online         No      No        6.0Gb/s     7.277TB  512E   Sup No  UP   V1GAWNFB WUS721208BLE604

4     0    4    2    SATA  HDD   member   online         No      No        6.0Gb/s     7.277TB  512E   Sup No  UP   V1GAWNFB WUS721208BLE604

5     0    5    7    SATA  HDD   member   online         No      No        6.0Gb/s     7.277TB  512E   Sup No  UP   V1GAWNFB WUS721208BLE604

6     0    6    8    SATA  HDD   member   online         No      No        6.0Gb/s     7.277TB  512E   Sup No  UP   V1GAWNFB WUS721208BLE604

7     0    7    11   SATA  HDD   rawdrive online         No      No        6.0Gb/s     7.277TB  512E   Sup No  UP   V1GAWNFB WUS721208BLE604

8     0    8    9    SATA  HDD   member   online         No      No        6.0Gb/s     7.277TB  512E   Sup No  UP   V1GAWNFB WUS721208BLE604

9     0    9    3    SATA  HDD   member   online         No      No        6.0Gb/s     7.277TB  512E   Sup No  UP   V1GAWNFB WUS721208BLE604

10    0    10   6    SATA  HDD   member   online         No      No        6.0Gb/s     7.277TB  512E   Sup No  UP   V1GAWNFB WUS721208BLE604

11    0    11   10   SATA  HDD   member   online         No      No        6.0Gb/s     7.277TB  512E   Sup No  UP   V1GAWNFB WUS721208BLE604

--------------------------------------------------------------------------------------------------------------

[root@bogon ~]# ./hiraidadm c0 show vdlist

========================================== Hiraidadm   Information ===========================================

Name: hiraidadm

Version: 1.2.2.9

Build Time: Jul  4 2025 11:03:05

==============================================================================================================

==============================================================================================================

Status Code = 0

Status = Success

Description = None

==============================================================================================================

Total vd number is 6

--------------------------------------------------------------------------------------------------------------

VDID RGID Name           Type    Status         Capacity   SUSz   SecSz  RCache   WCache RealWCache Access

--------------------------------------------------------------------------------------------------------------

0    0                   normal  normal          892.262GB   64KB   512B No_Ahead WT     WT         RW

1    1    RAID0_Disk2    normal  normal            7.276TB  256KB   512B No_Ahead WT     WT         RW

2    2    RAID1_Disk3_4  normal  normal            7.276TB  256KB   512B No_Ahead WT     WT         RW

3    3    RAID5_5_6_7    normal  normal           14.553TB  256KB   512B No_Ahead WT     WT         RW

4    4    RAID0_8        normal  normal            7.276TB  256KB   512B No_Ahead WT     WT         RW

5    5    RAID1_9_10     normal  normal            7.276TB  256KB   512B No_Ahead WT     WT         RW

--------------------------------------------------------------------------------------------------------------

[root@bogon ~]#